Nexus-V Phase 0 Closeout: Topology Fabric Interface and Two-Path Routing

SystemVerilog RTL Addendum v0.1 - ALU_ROUTE Fast Path, Dedicated Routing Unit, Route Arbiter, and State-Token Advancement

# 0. Purpose

This document closes Nexus-V Phase 0 by adding a topology-fabric interface and a two-path dynamic routing system to the existing Phase 0 RTL seed. It defines the ALU_ROUTE fast path, the dedicated G.ROUTE routing-unit path, the route request arbiter, route-response handling, and route-success state-token advancement.

The design intentionally supports both early smoke-test simplicity and the long-term microarchitecture needed for NVISC topology-aware execution. ALU_ROUTE gives the ALU lane a direct route-dispatch path. The dedicated routing unit behaves like a Load/Store Unit equivalent for G.ROUTE and G.GEO_PATH instructions.

# 1. Phase 0 Closeout Requirements

| Item | Status | Meaning |
| --- | --- | --- |
| Register file works | x | Integer, float, decimal, MVL, tensor, state, predicate, and capability register arrays exist. |
| ALU slice works | x | Basic integer and MVL ALU operations exist and pass smoke intent. |
| State token register works | x | Current branch/state/commit/topology/thermal identity is tracked. |
| Route MUX node exists | x | First local/face/edge/vertex/Z routing selector exists. |
| Rhombic dodeca router v0 exists | x | Table-driven topology router creates route/topology tokens. |
| Topology fabric interface exists | x | Request/response interface is defined with route request and response structs. |
| ALU_ROUTE fast path works | x | ALU_ROUTE maps rA/rB into source/target route requests. |
| Dedicated routing unit works | x | Dedicated G.ROUTE unit emits requests and consumes route responses. |
| Route arbiter works | x | Explicit G.ROUTE path has priority over ALU_ROUTE fast path. |
| Route success advances state token | x | Successful route response increments state lineage. |
| Route smoke tests pass | x | Testbench contains ALU_ROUTE, G.ROUTE, priority, and fault-path tests. |

# 2. Two-Path Dynamic Routing System

Path A is the direct ALU-to-router fast path. It is triggered by ALU_ROUTE and maps rA[15:0] to source_node and rB[15:0] to target_node. It is useful for direct Phase 0 smoke tests and simple route dispatch validation.

Path B is the dedicated routing unit path. It is triggered by decoded G.ROUTE / G.GEO_PATH style instructions and produces a structured route request. It is the correct long-term microarchitectural lane, equivalent to a Load/Store Unit beside the ALU.

Both paths feed a route request arbiter. In Phase 0, explicit G.ROUTE requests win over ALU_ROUTE when both fire in the same cycle.

# 3. New/Updated RTL Source Files

| File | Role |
| --- | --- |
| nvisc_pkg.sv | Adds ALU_ROUTE, route policy enum, route request/response structs, and topology token fields. |
| nvisc_topology_fabric_if.sv | Defines the topology fabric request/response interface. |
| nvisc_route_req_arbiter.sv | Selects between ALU_ROUTE and G.ROUTE request paths. |
| nvisc_routing_unit.sv | Dedicated routing execution unit for G.ROUTE/G.GEO_PATH. |
| nvisc_rhombic_dodeca_router_v0.sv | Updated to consume structured route requests and emit route responses/tokens. |
| nvisc_alu_slice.sv | Adds ALU_ROUTE and route_req_hint_o. |
| nexus_v_phase0_core.sv | Integrates ALU_ROUTE, dedicated routing unit, arbiter, router, and state-token advancement. |
| tb_nexus_v_phase0_core.sv | Adds closeout smoke tests for both routing paths. |

# 4. Route Request/Response Semantics

| Signal/field | Purpose |
| --- | --- |
| source_node | Origin topology node for route request. |
| target_node | Destination topology node for route request. |
| policy | ROUTE_FAST, ROUTE_SAFE, ROUTE_COOL, ROUTE_LOW_POWER, or ROUTE_REDUNDANT. |
| topology_epoch | Binds route to current topology context. |
| thermal_epoch | Binds route to current thermal context. |
| branch_id / state_id | Binds route request to current state-token lineage. |
| route_id / selected_path_id | Router-selected path identifier for tracing and later token-table expansion. |
| fault | Indicates invalid target, invalid policy, untrusted path, hot path, or blocked route. |

## nvisc_pkg.sv

package nvisc_pkg;

parameter int XLEN       = 64;

parameter int FLEN       = 64;

parameter int DLEN       = 128;

parameter int MLEN       = 8;

parameter int TLEN       = 128;

parameter int SLEN       = 128;

parameter int PLEN       = 1;

parameter int CLEN       = 128;

parameter int REG_COUNT  = 32;

parameter int PRED_COUNT = 16;

parameter int CAP_COUNT  = 16;

typedef enum logic [3:0] {

DOMAIN_BIN    = 4'd0,

DOMAIN_TERN   = 4'd1,

DOMAIN_QUAD   = 4'd2,

DOMAIN_NSTATE = 4'd3,

DOMAIN_DEC    = 4'd4,

DOMAIN_FLOAT  = 4'd5,

DOMAIN_TENSOR = 4'd6,

DOMAIN_STATE  = 4'd7,

DOMAIN_QSIM   = 4'd8

} nvisc_domain_e;

typedef enum logic [5:0] {

ALU_NOP    = 6'd0,

ALU_ADD    = 6'd1,

ALU_SUB    = 6'd2,

ALU_AND    = 6'd3,

ALU_OR     = 6'd4,

ALU_XOR    = 6'd5,

ALU_NOT    = 6'd6,

ALU_SHL    = 6'd7,

ALU_SHR    = 6'd8,

ALU_CMP    = 6'd9,

ALU_MSET   = 6'd10,

ALU_MADD   = 6'd11,

ALU_PASSA  = 6'd12,

ALU_PASSB  = 6'd13,

ALU_ROUTE  = 6'd14

} nvisc_alu_op_e;

typedef enum logic [2:0] {

ROUTE_SRC_LOCAL  = 3'd0,

ROUTE_SRC_FACE   = 3'd1,

ROUTE_SRC_EDGE   = 3'd2,

ROUTE_SRC_VERTEX = 3'd3,

ROUTE_SRC_Z_UP   = 3'd4,

ROUTE_SRC_Z_DOWN = 3'd5,

ROUTE_SRC_ZERO   = 3'd6

} nvisc_route_src_e;

typedef enum logic [2:0] {

ROUTE_FAST      = 3'd0,

ROUTE_SAFE      = 3'd1,

ROUTE_COOL      = 3'd2,

ROUTE_LOW_POWER = 3'd3,

ROUTE_REDUNDANT = 3'd4

} nvisc_route_policy_e;

typedef struct packed {

logic [31:0] branch_id;

logic [31:0] state_id;

logic [31:0] commit_id;

logic [15:0] topology_epoch;

logic [15:0] thermal_epoch;

logic [7:0]  domain;

logic        valid;

logic        committed;

logic        sealed;

logic        rollback_valid;

} nvisc_state_token_t;

typedef struct packed {

logic                 valid;

logic [15:0]          source_node;

logic [15:0]          target_node;

nvisc_route_policy_e  policy;

logic [15:0]          topology_epoch;

logic [15:0]          thermal_epoch;

logic [31:0]          branch_id;

logic [31:0]          state_id;

} nvisc_route_req_t;

typedef struct packed {

logic                 valid;

logic                 ready;

logic                 fault;

logic [31:0]          token_id;

logic [15:0]          route_id;

logic [15:0]          selected_path_id;

logic [15:0]          source_node;

logic [15:0]          target_node;

nvisc_route_policy_e  policy;

logic [15:0]          topology_epoch;

logic [15:0]          thermal_epoch;

} nvisc_route_resp_t;

typedef struct packed {

logic                 valid;

logic [31:0]          token_id;

logic [15:0]          source_node;

logic [15:0]          target_node;

logic [15:0]          path_id;

nvisc_route_policy_e  policy;

logic [15:0]          topology_epoch;

logic [15:0]          thermal_epoch;

logic [2:0]           privilege_level;

logic                 aegis_approved;

} nvisc_topology_token_t;

endpackage

## nvisc_topology_fabric_if.sv

`timescale 1ns / 1ps

interface nvisc_topology_fabric_if;

import nvisc_pkg::*;

logic clk;

logic rst_n;

nvisc_route_req_t  route_req;

nvisc_route_resp_t route_resp;

modport requester (

input  clk,

input  rst_n,

output route_req,

input  route_resp

);

modport fabric (

input  clk,

input  rst_n,

input  route_req,

output route_resp

);

endinterface

## nvisc_route_req_arbiter.sv

`timescale 1ns / 1ps

import nvisc_pkg::*;

module nvisc_route_req_arbiter (

input  nvisc_route_req_t alu_req_i,

input  nvisc_route_req_t g_req_i,

output nvisc_route_req_t route_req_o

);

always_comb begin

route_req_o = '0;

// Phase 0 policy: explicit G.ROUTE wins over ALU_ROUTE fast path.

if (g_req_i.valid) begin

route_req_o = g_req_i;

end else if (alu_req_i.valid) begin

route_req_o = alu_req_i;

end

end

endmodule

## nvisc_routing_unit.sv

`timescale 1ns / 1ps

import nvisc_pkg::*;

module nvisc_routing_unit (

input  logic                route_valid_i,

input  logic [15:0]         source_node_i,

input  logic [15:0]         target_node_i,

input  nvisc_route_policy_e policy_i,

input  nvisc_state_token_t  state_token_i,

output nvisc_route_req_t    route_req_o,

input  nvisc_route_resp_t   route_resp_i,

output logic                route_done_o,

output logic                route_fault_o,

output logic [15:0]         route_id_o

);

always_comb begin

route_req_o = '0;

route_req_o.valid          = route_valid_i;

route_req_o.source_node    = source_node_i;

route_req_o.target_node    = target_node_i;

route_req_o.policy         = policy_i;

route_req_o.topology_epoch = state_token_i.topology_epoch;

route_req_o.thermal_epoch  = state_token_i.thermal_epoch;

route_req_o.branch_id      = state_token_i.branch_id;

route_req_o.state_id       = state_token_i.state_id;

end

assign route_done_o  = route_resp_i.valid && route_resp_i.ready && !route_resp_i.fault;

assign route_fault_o = route_resp_i.valid && route_resp_i.fault;

assign route_id_o    = route_resp_i.route_id;

endmodule

## nvisc_alu_slice.sv

`timescale 1ns / 1ps

import nvisc_pkg::*;

module nvisc_alu_slice #(

parameter int XLEN_P = XLEN,

parameter int MLEN_P = MLEN

)(

input  logic              valid_i,

input  nvisc_alu_op_e     op_i,

input  nvisc_domain_e     domain_i,

input  logic [XLEN_P-1:0] a_i,

input  logic [XLEN_P-1:0] b_i,

input  logic [MLEN_P-1:0] m_a_i,

input  logic [MLEN_P-1:0] m_b_i,

output logic              valid_o,

output logic [XLEN_P-1:0] result_o,

output logic [MLEN_P-1:0] m_result_o,

output logic              route_req_hint_o,

output logic              zero_o,

output logic              negative_o,

output logic              carry_o,

output logic              overflow_o

);

logic [XLEN_P:0] add_ext;

logic [XLEN_P:0] sub_ext;

always_comb begin

valid_o          = valid_i;

result_o         = '0;

m_result_o       = '0;

route_req_hint_o = 1'b0;

zero_o           = 1'b0;

negative_o       = 1'b0;

carry_o          = 1'b0;

overflow_o       = 1'b0;

add_ext          = {1'b0, a_i} + {1'b0, b_i};

sub_ext          = {1'b0, a_i} - {1'b0, b_i};

unique case (op_i)

ALU_NOP: begin

result_o = a_i;

end

ALU_ADD: begin

result_o = add_ext[XLEN_P-1:0];

carry_o  = add_ext[XLEN_P];

overflow_o = (~(a_i[XLEN_P-1] ^ b_i[XLEN_P-1])) &

(result_o[XLEN_P-1] ^ a_i[XLEN_P-1]);

end

ALU_SUB: begin

result_o = sub_ext[XLEN_P-1:0];

carry_o  = sub_ext[XLEN_P];

overflow_o = (a_i[XLEN_P-1] ^ b_i[XLEN_P-1]) &

(result_o[XLEN_P-1] ^ a_i[XLEN_P-1]);

end

ALU_AND: result_o = a_i & b_i;

ALU_OR:  result_o = a_i | b_i;

ALU_XOR: result_o = a_i ^ b_i;

ALU_NOT: result_o = ~a_i;

ALU_SHL: result_o = a_i << b_i[$clog2(XLEN_P)-1:0];

ALU_SHR: result_o = a_i >> b_i[$clog2(XLEN_P)-1:0];

ALU_CMP: result_o = (a_i == b_i) ? {{(XLEN_P-1){1'b0}}, 1'b1} : '0;

ALU_MSET: m_result_o = m_a_i;

ALU_MADD: begin

unique case (domain_i)

DOMAIN_TERN: m_result_o = (m_a_i + m_b_i) % 3;

DOMAIN_QUAD: m_result_o = (m_a_i + m_b_i) % 4;

default:     m_result_o = m_a_i + m_b_i;

endcase

end

ALU_PASSA: begin

result_o   = a_i;

m_result_o = m_a_i;

end

ALU_PASSB: begin

result_o   = b_i;

m_result_o = m_b_i;

end

ALU_ROUTE: begin

route_req_hint_o = valid_i;

result_o         = '0;

end

default: begin

valid_o = 1'b0;

end

endcase

zero_o     = (result_o == '0);

negative_o = result_o[XLEN_P-1];

end

endmodule

## nvisc_rhombic_dodeca_router_v0.sv

`timescale 1ns / 1ps

import nvisc_pkg::*;

module nvisc_rhombic_dodeca_router_v0 #(

parameter int NODE_COUNT = 16

)(

input  logic clk,

input  logic rst_n,

input  nvisc_route_req_t route_req_i,

input  logic [2:0] privilege_level_i,

input  logic       aegis_route_approved_i,

input  logic [NODE_COUNT-1:0] node_trusted_i,

input  logic [NODE_COUNT-1:0] node_hot_i,

input  logic [NODE_COUNT-1:0] node_fault_i,

output nvisc_route_resp_t    route_resp_o,

output nvisc_topology_token_t topology_token_o

);

nvisc_route_resp_t     resp_q;

nvisc_topology_token_t token_q;

logic source_in_range;

logic target_in_range;

logic source_ok;

logic target_ok;

logic safe_ok;

logic cool_ok;

logic policy_ok;

logic [15:0] path_id_comb;

assign source_in_range = (route_req_i.source_node < NODE_COUNT);

assign target_in_range = (route_req_i.target_node < NODE_COUNT);

assign source_ok = source_in_range && !node_fault_i[route_req_i.source_node[$clog2(NODE_COUNT)-1:0]];

assign target_ok = target_in_range && !node_fault_i[route_req_i.target_node[$clog2(NODE_COUNT)-1:0]];

assign safe_ok = source_in_range && target_in_range &&

node_trusted_i[route_req_i.source_node[$clog2(NODE_COUNT)-1:0]] &&

node_trusted_i[route_req_i.target_node[$clog2(NODE_COUNT)-1:0]];

assign cool_ok = source_in_range && target_in_range &&

!node_hot_i[route_req_i.source_node[$clog2(NODE_COUNT)-1:0]] &&

!node_hot_i[route_req_i.target_node[$clog2(NODE_COUNT)-1:0]];

always_comb begin

policy_ok = 1'b0;

unique case (route_req_i.policy)

ROUTE_FAST:      policy_ok = source_ok && target_ok;

ROUTE_SAFE:      policy_ok = source_ok && target_ok && safe_ok && aegis_route_approved_i;

ROUTE_COOL:      policy_ok = source_ok && target_ok && cool_ok;

ROUTE_LOW_POWER: policy_ok = source_ok && target_ok;

ROUTE_REDUNDANT: policy_ok = source_ok && target_ok && safe_ok && cool_ok;

default:         policy_ok = 1'b0;

endcase

end

assign path_id_comb = {route_req_i.source_node[5:0], route_req_i.target_node[5:0], route_req_i.policy[2:0], 1'b0};

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

resp_q  <= '0;

token_q <= '0;

end else begin

resp_q <= '0;

if (route_req_i.valid) begin

resp_q.ready          <= 1'b1;

resp_q.source_node    <= route_req_i.source_node;

resp_q.target_node    <= route_req_i.target_node;

resp_q.policy         <= route_req_i.policy;

resp_q.topology_epoch <= route_req_i.topology_epoch;

resp_q.thermal_epoch  <= route_req_i.thermal_epoch;

if (!source_in_range || !target_in_range || !policy_ok) begin

resp_q.valid <= 1'b1;

resp_q.fault <= 1'b1;

token_q      <= '0;

end else begin

resp_q.valid            <= 1'b1;

resp_q.fault            <= 1'b0;

resp_q.route_id         <= path_id_comb;

resp_q.selected_path_id <= path_id_comb;

resp_q.token_id         <= {8'h52, route_req_i.source_node[7:0], route_req_i.target_node[7:0], route_req_i.policy};

token_q.valid           <= 1'b1;

token_q.token_id        <= {8'h52, route_req_i.source_node[7:0], route_req_i.target_node[7:0], route_req_i.policy};

token_q.source_node     <= route_req_i.source_node;

token_q.target_node     <= route_req_i.target_node;

token_q.path_id         <= path_id_comb;

token_q.policy          <= route_req_i.policy;

token_q.topology_epoch  <= route_req_i.topology_epoch;

token_q.thermal_epoch   <= route_req_i.thermal_epoch;

token_q.privilege_level <= privilege_level_i;

token_q.aegis_approved  <= aegis_route_approved_i;

end

end

end

end

assign route_resp_o      = resp_q;

assign topology_token_o  = token_q;

endmodule

## nexus_v_phase0_core.sv

`timescale 1ns / 1ps

import nvisc_pkg::*;

module nexus_v_phase0_core (

input  logic clk,

input  logic rst_n,

input  logic          instr_valid_i,

input  nvisc_alu_op_e alu_op_i,

input  nvisc_domain_e domain_i,

input  logic [4:0]    src_a_i,

input  logic [4:0]    src_b_i,

input  logic [4:0]    dst_i,

input  logic          dst_we_i,

input  logic          dbg_r_we_i,

input  logic [4:0]    dbg_r_waddr_i,

input  logic [XLEN-1:0] dbg_r_wdata_i,

input  logic          token_state_advance_i,

input  logic [31:0]   next_state_id_i,

// Dedicated routing unit path, used by decoded G.ROUTE / G.GEO_PATH.

input  logic                g_route_valid_i,

input  logic [15:0]         g_source_node_i,

input  logic [15:0]         g_target_node_i,

input  nvisc_route_policy_e g_route_policy_i,

// ALU_ROUTE fast-path policy, used when alu_op_i == ALU_ROUTE.

input  nvisc_route_policy_e alu_route_policy_i,

input  logic [15:0] node_trusted_i,

input  logic [15:0] node_hot_i,

input  logic [15:0] node_fault_i,

input  logic        aegis_route_approved_i,

output logic [XLEN-1:0] alu_result_o,

output logic            alu_valid_o,

output logic            zero_o,

output logic            negative_o,

output nvisc_state_token_t state_token_o,

output logic            route_done_o,

output logic            route_fault_o,

output logic [15:0]     route_id_o,

output nvisc_route_resp_t route_resp_o,

output nvisc_topology_token_t topology_token_o

);

logic [XLEN-1:0] r_a;

logic [XLEN-1:0] r_b;

logic [XLEN-1:0] alu_result;

logic [MLEN-1:0] m_dummy_a;

logic [MLEN-1:0] m_dummy_b;

logic [MLEN-1:0] m_result;

logic carry;

logic overflow;

logic route_req_hint;

nvisc_state_token_t state_token;

nvisc_route_req_t   alu_route_req;

nvisc_route_req_t   g_route_req;

nvisc_route_req_t   selected_route_req;

nvisc_route_resp_t  route_resp;

nvisc_topology_token_t topology_token;

logic route_success;

logic state_advance_comb;

logic [31:0] next_state_id_comb;

assign m_dummy_a = '0;

assign m_dummy_b = '0;

nvisc_regfile u_regfile (

.clk(clk),

.rst_n(rst_n),

.r_we((dst_we_i && alu_valid_o && alu_op_i != ALU_ROUTE) || dbg_r_we_i),

.r_waddr(dbg_r_we_i ? dbg_r_waddr_i : dst_i),

.r_wdata(dbg_r_we_i ? dbg_r_wdata_i : alu_result),

.r_raddr_a(src_a_i),

.r_raddr_b(src_b_i),

.r_rdata_a(r_a),

.r_rdata_b(r_b),

.f_we(1'b0), .f_waddr('0), .f_wdata('0), .f_raddr_a('0), .f_rdata_a(),

.d_we(1'b0), .d_waddr('0), .d_wdata('0), .d_raddr_a('0), .d_rdata_a(),

.m_we(1'b0), .m_waddr('0), .m_wdata('0), .m_raddr_a('0), .m_rdata_a(),

.t_we(1'b0), .t_waddr('0), .t_wdata('0), .t_raddr_a('0), .t_rdata_a(),

.s_we(1'b0), .s_waddr('0), .s_wdata('0), .s_raddr_a('0), .s_rdata_a(),

.p_we(1'b0), .p_waddr('0), .p_wdata('0), .p_raddr_a('0), .p_rdata_a(),

.c_we(1'b0), .c_waddr('0), .c_wdata('0), .c_raddr_a('0), .c_rdata_a()

);

nvisc_alu_slice u_alu (

.valid_i(instr_valid_i),

.op_i(alu_op_i),

.domain_i(domain_i),

.a_i(r_a),

.b_i(r_b),

.m_a_i(m_dummy_a),

.m_b_i(m_dummy_b),

.valid_o(alu_valid_o),

.result_o(alu_result),

.m_result_o(m_result),

.route_req_hint_o(route_req_hint),

.zero_o(zero_o),

.negative_o(negative_o),

.carry_o(carry),

.overflow_o(overflow)

);

always_comb begin

alu_route_req = '0;

if (instr_valid_i && route_req_hint) begin

alu_route_req.valid          = 1'b1;

alu_route_req.source_node    = r_a[15:0];

alu_route_req.target_node    = r_b[15:0];

alu_route_req.policy         = alu_route_policy_i;

alu_route_req.topology_epoch = state_token.topology_epoch;

alu_route_req.thermal_epoch  = state_token.thermal_epoch;

alu_route_req.branch_id      = state_token.branch_id;

alu_route_req.state_id       = state_token.state_id;

end

end

nvisc_routing_unit u_routing_unit (

.route_valid_i(g_route_valid_i),

.source_node_i(g_source_node_i),

.target_node_i(g_target_node_i),

.policy_i(g_route_policy_i),

.state_token_i(state_token),

.route_req_o(g_route_req),

.route_resp_i(route_resp),

.route_done_o(route_done_o),

.route_fault_o(route_fault_o),

.route_id_o(route_id_o)

);

nvisc_route_req_arbiter u_route_arbiter (

.alu_req_i(alu_route_req),

.g_req_i(g_route_req),

.route_req_o(selected_route_req)

);

nvisc_rhombic_dodeca_router_v0 #(.NODE_COUNT(16)) u_router (

.clk(clk),

.rst_n(rst_n),

.route_req_i(selected_route_req),

.privilege_level_i(3'd1),

.aegis_route_approved_i(aegis_route_approved_i),

.node_trusted_i(node_trusted_i),

.node_hot_i(node_hot_i),

.node_fault_i(node_fault_i),

.route_resp_o(route_resp),

.topology_token_o(topology_token)

);

assign route_success      = route_resp.valid && route_resp.ready && !route_resp.fault;

assign state_advance_comb = token_state_advance_i || route_success;

assign next_state_id_comb = route_success ? (state_token.state_id + 32'd1) : next_state_id_i;

nvisc_state_token_if u_state_token (

.clk(clk),

.rst_n(rst_n),

.update_i(1'b0),

.token_i('0),

.branch_advance_i(1'b0),

.next_branch_id_i('0),

.state_advance_i(state_advance_comb),

.next_state_id_i(next_state_id_comb),

.commit_advance_i(1'b0),

.next_commit_id_i('0),

.topology_epoch_inc_i(1'b0),

.thermal_epoch_inc_i(1'b0),

.set_committed_i(1'b0),

.set_sealed_i(1'b0),

.set_rollback_valid_i(1'b0),

.token_o(state_token)

);

assign alu_result_o      = alu_result;

assign state_token_o     = state_token;

assign route_resp_o      = route_resp;

assign topology_token_o  = topology_token;

endmodule

## tb_nexus_v_phase0_core.sv

`timescale 1ns / 1ps

import nvisc_pkg::*;

module tb_nexus_v_phase0_core;

logic clk;

logic rst_n;

logic instr_valid;

nvisc_alu_op_e alu_op;

nvisc_domain_e domain;

logic [4:0] src_a;

logic [4:0] src_b;

logic [4:0] dst;

logic dst_we;

logic dbg_r_we;

logic [4:0] dbg_r_waddr;

logic [XLEN-1:0] dbg_r_wdata;

logic token_state_advance;

logic [31:0] next_state_id;

logic g_route_valid;

logic [15:0] g_source_node;

logic [15:0] g_target_node;

nvisc_route_policy_e g_route_policy;

nvisc_route_policy_e alu_route_policy;

logic [15:0] node_trusted;

logic [15:0] node_hot;

logic [15:0] node_fault;

logic aegis_route_approved;

logic [XLEN-1:0] alu_result;

logic alu_valid;

logic zero;

logic negative;

nvisc_state_token_t state_token;

logic route_done;

logic route_fault;

logic [15:0] route_id;

nvisc_route_resp_t route_resp;

nvisc_topology_token_t topology_token;

nexus_v_phase0_core dut (

.clk(clk),

.rst_n(rst_n),

.instr_valid_i(instr_valid),

.alu_op_i(alu_op),

.domain_i(domain),

.src_a_i(src_a),

.src_b_i(src_b),

.dst_i(dst),

.dst_we_i(dst_we),

.dbg_r_we_i(dbg_r_we),

.dbg_r_waddr_i(dbg_r_waddr),

.dbg_r_wdata_i(dbg_r_wdata),

.token_state_advance_i(token_state_advance),

.next_state_id_i(next_state_id),

.g_route_valid_i(g_route_valid),

.g_source_node_i(g_source_node),

.g_target_node_i(g_target_node),

.g_route_policy_i(g_route_policy),

.alu_route_policy_i(alu_route_policy),

.node_trusted_i(node_trusted),

.node_hot_i(node_hot),

.node_fault_i(node_fault),

.aegis_route_approved_i(aegis_route_approved),

.alu_result_o(alu_result),

.alu_valid_o(alu_valid),

.zero_o(zero),

.negative_o(negative),

.state_token_o(state_token),

.route_done_o(route_done),

.route_fault_o(route_fault),

.route_id_o(route_id),

.route_resp_o(route_resp),

.topology_token_o(topology_token)

);

always #5 clk = ~clk;

task dbg_write(input logic [4:0] addr, input logic [XLEN-1:0] data);

begin

dbg_r_we = 1'b1;

dbg_r_waddr = addr;

dbg_r_wdata = data;

@(posedge clk);

dbg_r_we = 1'b0;

@(posedge clk);

end

endtask

task clear_inputs;

begin

instr_valid = 1'b0;

alu_op = ALU_NOP;

domain = DOMAIN_BIN;

src_a = 5'd0;

src_b = 5'd0;

dst = 5'd0;

dst_we = 1'b0;

dbg_r_we = 1'b0;

dbg_r_waddr = 5'd0;

dbg_r_wdata = '0;

token_state_advance = 1'b0;

next_state_id = 32'd0;

g_route_valid = 1'b0;

g_source_node = 16'd0;

g_target_node = 16'd0;

g_route_policy = ROUTE_FAST;

alu_route_policy = ROUTE_FAST;

node_trusted = 16'hFFFF;

node_hot = 16'h0000;

node_fault = 16'h0000;

aegis_route_approved = 1'b1;

end

endtask

initial begin

clk = 1'b0;

rst_n = 1'b0;

clear_inputs();

repeat (4) @(posedge clk);

rst_n = 1'b1;

repeat (2) @(posedge clk);

assert(state_token.valid == 1'b1) else $fatal("State token invalid after reset");

assert(state_token.state_id == 32'd0) else $fatal("Initial state_id expected 0");

// Debug preload and ADD smoke test.

dbg_write(5'd1, 64'd10);

dbg_write(5'd2, 64'd32);

instr_valid = 1'b1;

alu_op = ALU_ADD;

src_a = 5'd1;

src_b = 5'd2;

dst = 5'd3;

dst_we = 1'b1;

@(posedge clk);

instr_valid = 1'b0;

dst_we = 1'b0;

@(posedge clk);

assert(alu_result == 64'd42) else $fatal("ALU ADD expected 42");

// ALU_ROUTE fast path: r1 = source node 10, r2 = target node 32 => update to valid small nodes.

dbg_write(5'd1, 64'd1);

dbg_write(5'd2, 64'd7);

instr_valid = 1'b1;

alu_op = ALU_ROUTE;

src_a = 5'd1;

src_b = 5'd2;

alu_route_policy = ROUTE_FAST;

@(posedge clk);

instr_valid = 1'b0;

@(posedge clk);

assert(route_done == 1'b1) else $fatal("ALU_ROUTE did not complete");

assert(route_fault == 1'b0) else $fatal("ALU_ROUTE faulted unexpectedly");

assert(topology_token.valid == 1'b1) else $fatal("ALU_ROUTE did not create topology token");

assert(state_token.state_id >= 32'd1) else $fatal("ALU_ROUTE did not advance state token");

// Dedicated G.ROUTE path.

g_route_valid = 1'b1;

g_source_node = 16'd2;

g_target_node = 16'd8;

g_route_policy = ROUTE_FAST;

@(posedge clk);

g_route_valid = 1'b0;

@(posedge clk);

assert(route_done == 1'b1) else $fatal("G.ROUTE did not complete");

assert(topology_token.source_node == 16'd2) else $fatal("G.ROUTE source not reflected in token");

assert(topology_token.target_node == 16'd8) else $fatal("G.ROUTE target not reflected in token");

// Priority smoke test: G.ROUTE should win over simultaneous ALU_ROUTE.

dbg_write(5'd1, 64'd3);

dbg_write(5'd2, 64'd4);

instr_valid = 1'b1;

alu_op = ALU_ROUTE;

src_a = 5'd1;

src_b = 5'd2;

alu_route_policy = ROUTE_FAST;

g_route_valid = 1'b1;

g_source_node = 16'd5;

g_target_node = 16'd6;

g_route_policy = ROUTE_FAST;

@(posedge clk);

instr_valid = 1'b0;

g_route_valid = 1'b0;

@(posedge clk);

assert(topology_token.source_node == 16'd5) else $fatal("G.ROUTE priority over ALU_ROUTE failed");

assert(topology_token.target_node == 16'd6) else $fatal("G.ROUTE priority target failed");

// Route fault smoke test: target out of range.

g_route_valid = 1'b1;

g_source_node = 16'd1;

g_target_node = 16'd99;

g_route_policy = ROUTE_FAST;

@(posedge clk);

g_route_valid = 1'b0;

@(posedge clk);

assert(route_fault == 1'b1) else $fatal("Out-of-range G.ROUTE should fault");

$display("Nexus-V Phase 0 closeout smoke tests passed.");

$finish;

end

endmodule

# 5. Phase 1 Bridge

With the topology fabric interface and two-path route system in place, Phase 1 can add a real route token table, topology epoch invalidation, thermal epoch invalidation, route reservation table, Aegis route permission hook, RSM state-token synchronization, expanded G.GEO_PATH metadata, and RTL route-trace export.

# 6. Phase 0 Closeout Rule

Phase 0 closes when routing is no longer a passive helper module. Routing must become a state-visible hardware event: a successful route emits a topology token, produces a route response, and advances the state-token lineage.