Nexus-V Phase 0 SystemVerilog RTL Modules

NVISC Register File, ALU Slice, State Token Interface, Route MUX Node, and Rhombic-Dodeca Router v0

Status: implementation seed documentation for Phase 0 RTL smoke tests and the next route-fabric module step. This document turns the Nexus-V RTL microarchitecture direction into concrete SystemVerilog module boundaries and starter code.

# 1. Purpose and Phase 0 Scope

Phase 0 creates a minimal synthesizable SystemVerilog seed for Nexus-V. It is not the complete NVISC processor; it is the smallest hardware substrate that proves the register banks, ALU slice, state-token register, and first 3D route-fabric abstractions can exist as RTL.

| Track | Phase 0 hardware deliverable | Reason |
| --- | --- | --- |
| NVISC register file | Register arrays for r/f/d/m/t/s/p/c classes | Preserves the unique NVISC register model, including state and tensor handles. |
| ALU slice | Basic integer and MVL operations | Provides smoke-test arithmetic and first Volumetric ALU slice. |
| State token interface | Branch/state/commit/topology/thermal identity register | Provides hardware identity for versioned state and future RSM linkage. |
| Route MUX node | Parameterized local/face/edge/vertex/Z source mux | First physical seed of 3D route data movement. |
| Rhombic-dodeca router v0 | Table/metadata-driven route-token issuer | Functional placeholder for future polyhedral routing hardware. |

# 2. File Layout

nexus_v_phase0/

├─ rtl/

│  ├─ nvisc_pkg.sv

│  ├─ nvisc_regfile.sv

│  ├─ nvisc_alu_slice.sv

│  ├─ nvisc_state_token_if.sv

│  ├─ nvisc_route_mux_node.sv

│  ├─ nvisc_rhombic_dodeca_router_v0.sv

│  └─ nexus_v_phase0_core.sv

└─ tb/

└─ tb_nexus_v_phase0_core.sv

# 3. Design Assumptions

All Phase 0 modules should stay synthesizable and simple enough for FPGA smoke tests.

The rhombic-dodecahedron router is represented as route metadata and route tokens first, not full physical 3D silicon.

Register r0 is hardwired to zero in profiles that enable ZERO_R0 behavior.

Tensor t registers and state s registers are handles at this stage, not full tensor/state storage arrays.

The route fabric is token-oriented: a successful route creates a topology token that later modules can audit, reserve, expire, or consume.

The state token carries branch/state/commit lineage and topology/thermal epochs so route handles can later expire correctly.

# 4. Package Definitions: nvisc_pkg.sv

Defines global widths, domains, ALU opcodes, route source selectors, route policies, state-token structure, and topology-token structure.

package nvisc_pkg;

parameter int XLEN       = 64;

parameter int FLEN       = 64;

parameter int DLEN       = 128;

parameter int MLEN       = 8;

parameter int TLEN       = 128;

parameter int SLEN       = 128;

parameter int PLEN       = 1;

parameter int CLEN       = 128;

parameter int REG_COUNT  = 32;

parameter int PRED_COUNT = 16;

parameter int CAP_COUNT  = 16;

typedef enum logic [3:0] {

DOMAIN_BIN    = 4'd0,

DOMAIN_TERN   = 4'd1,

DOMAIN_QUAD   = 4'd2,

DOMAIN_NSTATE = 4'd3,

DOMAIN_DEC    = 4'd4,

DOMAIN_FLOAT  = 4'd5,

DOMAIN_TENSOR = 4'd6,

DOMAIN_STATE  = 4'd7,

DOMAIN_QSIM   = 4'd8

} nvisc_domain_e;

typedef enum logic [5:0] {

ALU_NOP   = 6'd0,

ALU_ADD   = 6'd1,

ALU_SUB   = 6'd2,

ALU_AND   = 6'd3,

ALU_OR    = 6'd4,

ALU_XOR   = 6'd5,

ALU_NOT   = 6'd6,

ALU_SHL   = 6'd7,

ALU_SHR   = 6'd8,

ALU_CMP   = 6'd9,

ALU_MSET  = 6'd10,

ALU_MADD  = 6'd11,

ALU_PASSA = 6'd12,

ALU_PASSB = 6'd13

} nvisc_alu_op_e;

typedef enum logic [2:0] {

ROUTE_SRC_LOCAL  = 3'd0,

ROUTE_SRC_FACE   = 3'd1,

ROUTE_SRC_EDGE   = 3'd2,

ROUTE_SRC_VERTEX = 3'd3,

ROUTE_SRC_Z_UP   = 3'd4,

ROUTE_SRC_Z_DOWN = 3'd5,

ROUTE_SRC_ZERO   = 3'd6

} nvisc_route_src_e;

typedef enum logic [2:0] {

ROUTE_FAST      = 3'd0,

ROUTE_SAFE      = 3'd1,

ROUTE_COOL      = 3'd2,

ROUTE_LOW_POWER = 3'd3,

ROUTE_REDUNDANT = 3'd4

} nvisc_route_policy_e;

typedef struct packed {

logic [31:0] branch_id;

logic [31:0] state_id;

logic [31:0] commit_id;

logic [15:0] topology_epoch;

logic [15:0] thermal_epoch;

logic [7:0]  domain;

logic        valid;

logic        committed;

logic        sealed;

logic        rollback_valid;

} nvisc_state_token_t;

typedef struct packed {

logic        valid;

logic [31:0] token_id;

logic [7:0]  source_node;

logic [7:0]  target_node;

logic [15:0] path_id;

nvisc_route_policy_e policy;

logic [15:0] topology_epoch;

logic [15:0] thermal_epoch;

logic [2:0]  privilege_level;

logic        aegis_approved;

} nvisc_topology_token_t;

endpackage

# 5. NVISC Register File: nvisc_regfile.sv

Implements Phase 0 banks for integer, floating, decimal, multi-valued, tensor, state, predicate, and capability registers.

`timescale 1ns / 1ps

import nvisc_pkg::*;

module nvisc_regfile #(

parameter int XLEN_P = XLEN,

parameter int FLEN_P = FLEN,

parameter int DLEN_P = DLEN,

parameter int MLEN_P = MLEN,

parameter int TLEN_P = TLEN,

parameter int SLEN_P = SLEN,

parameter int CLEN_P = CLEN

)(

input  logic clk,

input  logic rst_n,

input  logic        r_we,

input  logic [4:0]  r_waddr,

input  logic [XLEN_P-1:0] r_wdata,

input  logic [4:0]  r_raddr_a,

input  logic [4:0]  r_raddr_b,

output logic [XLEN_P-1:0] r_rdata_a,

output logic [XLEN_P-1:0] r_rdata_b,

input  logic        f_we,

input  logic [4:0]  f_waddr,

input  logic [FLEN_P-1:0] f_wdata,

input  logic [4:0]  f_raddr_a,

output logic [FLEN_P-1:0] f_rdata_a,

input  logic        d_we,

input  logic [4:0]  d_waddr,

input  logic [DLEN_P-1:0] d_wdata,

input  logic [4:0]  d_raddr_a,

output logic [DLEN_P-1:0] d_rdata_a,

input  logic        m_we,

input  logic [4:0]  m_waddr,

input  logic [MLEN_P-1:0] m_wdata,

input  logic [4:0]  m_raddr_a,

output logic [MLEN_P-1:0] m_rdata_a,

input  logic        t_we,

input  logic [4:0]  t_waddr,

input  logic [TLEN_P-1:0] t_wdata,

input  logic [4:0]  t_raddr_a,

output logic [TLEN_P-1:0] t_rdata_a,

input  logic        s_we,

input  logic [4:0]  s_waddr,

input  logic [SLEN_P-1:0] s_wdata,

input  logic [4:0]  s_raddr_a,

output logic [SLEN_P-1:0] s_rdata_a,

input  logic        p_we,

input  logic [3:0]  p_waddr,

input  logic        p_wdata,

input  logic [3:0]  p_raddr_a,

output logic        p_rdata_a,

input  logic        c_we,

input  logic [3:0]  c_waddr,

input  logic [CLEN_P-1:0] c_wdata,

input  logic [3:0]  c_raddr_a,

output logic [CLEN_P-1:0] c_rdata_a

);

logic [XLEN_P-1:0] r_file [REG_COUNT];

logic [FLEN_P-1:0] f_file [REG_COUNT];

logic [DLEN_P-1:0] d_file [REG_COUNT];

logic [MLEN_P-1:0] m_file [REG_COUNT];

logic [TLEN_P-1:0] t_file [REG_COUNT];

logic [SLEN_P-1:0] s_file [REG_COUNT];

logic              p_file [PRED_COUNT];

logic [CLEN_P-1:0] c_file [CAP_COUNT];

integer i;

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

for (i = 0; i < REG_COUNT; i++) begin

r_file[i] <= '0;

f_file[i] <= '0;

d_file[i] <= '0;

m_file[i] <= '0;

t_file[i] <= '0;

s_file[i] <= '0;

end

for (i = 0; i < PRED_COUNT; i++) p_file[i] <= 1'b0;

for (i = 0; i < CAP_COUNT; i++) c_file[i] <= '0;

end else begin

if (r_we && r_waddr != 5'd0) r_file[r_waddr] <= r_wdata;

if (f_we) f_file[f_waddr] <= f_wdata;

if (d_we) d_file[d_waddr] <= d_wdata;

if (m_we) m_file[m_waddr] <= m_wdata;

if (t_we) t_file[t_waddr] <= t_wdata;

if (s_we) s_file[s_waddr] <= s_wdata;

if (p_we) p_file[p_waddr] <= p_wdata;

if (c_we) c_file[c_waddr] <= c_wdata;

end

end

assign r_rdata_a = (r_raddr_a == 5'd0) ? '0 : r_file[r_raddr_a];

assign r_rdata_b = (r_raddr_b == 5'd0) ? '0 : r_file[r_raddr_b];

assign f_rdata_a = f_file[f_raddr_a];

assign d_rdata_a = d_file[d_raddr_a];

assign m_rdata_a = m_file[m_raddr_a];

assign t_rdata_a = t_file[t_raddr_a];

assign s_rdata_a = s_file[s_raddr_a];

assign p_rdata_a = p_file[p_raddr_a];

assign c_rdata_a = c_file[c_raddr_a];

endmodule

# 6. ALU Slice: nvisc_alu_slice.sv

Implements the first scalar Volumetric ALU slice with integer arithmetic, bitwise logic, comparison, and basic MVL wraparound operations.

`timescale 1ns / 1ps

import nvisc_pkg::*;

module nvisc_alu_slice #(

parameter int XLEN_P = XLEN,

parameter int MLEN_P = MLEN

)(

input  logic              valid_i,

input  nvisc_alu_op_e     op_i,

input  nvisc_domain_e     domain_i,

input  logic [XLEN_P-1:0] a_i,

input  logic [XLEN_P-1:0] b_i,

input  logic [MLEN_P-1:0] m_a_i,

input  logic [MLEN_P-1:0] m_b_i,

output logic              valid_o,

output logic [XLEN_P-1:0] result_o,

output logic [MLEN_P-1:0] m_result_o,

output logic              zero_o,

output logic              negative_o,

output logic              carry_o,

output logic              overflow_o

);

logic [XLEN_P:0] add_ext;

logic [XLEN_P:0] sub_ext;

always_comb begin

valid_o    = valid_i;

result_o   = '0;

m_result_o = '0;

zero_o     = 1'b0;

negative_o = 1'b0;

carry_o    = 1'b0;

overflow_o = 1'b0;

add_ext    = {1'b0, a_i} + {1'b0, b_i};

sub_ext    = {1'b0, a_i} - {1'b0, b_i};

unique case (op_i)

ALU_NOP: result_o = a_i;

ALU_ADD: begin

result_o = add_ext[XLEN_P-1:0];

carry_o  = add_ext[XLEN_P];

overflow_o = (~(a_i[XLEN_P-1] ^ b_i[XLEN_P-1])) &

(result_o[XLEN_P-1] ^ a_i[XLEN_P-1]);

end

ALU_SUB: begin

result_o = sub_ext[XLEN_P-1:0];

carry_o  = sub_ext[XLEN_P];

overflow_o = (a_i[XLEN_P-1] ^ b_i[XLEN_P-1]) &

(result_o[XLEN_P-1] ^ a_i[XLEN_P-1]);

end

ALU_AND: result_o = a_i & b_i;

ALU_OR:  result_o = a_i | b_i;

ALU_XOR: result_o = a_i ^ b_i;

ALU_NOT: result_o = ~a_i;

ALU_SHL: result_o = a_i << b_i[$clog2(XLEN_P)-1:0];

ALU_SHR: result_o = a_i >> b_i[$clog2(XLEN_P)-1:0];

ALU_CMP: result_o = (a_i == b_i) ? {{(XLEN_P-1){1'b0}}, 1'b1} : '0;

ALU_MSET: m_result_o = m_a_i;

ALU_MADD: begin

unique case (domain_i)

DOMAIN_TERN: m_result_o = (m_a_i + m_b_i) % 3;

DOMAIN_QUAD: m_result_o = (m_a_i + m_b_i) % 4;

default:     m_result_o = m_a_i + m_b_i;

endcase

end

ALU_PASSA: begin result_o = a_i; m_result_o = m_a_i; end

ALU_PASSB: begin result_o = b_i; m_result_o = m_b_i; end

default: valid_o = 1'b0;

endcase

zero_o     = (result_o == '0);

negative_o = result_o[XLEN_P-1];

end

endmodule

# 7. State Token Register Interface: nvisc_state_token_if.sv

Tracks the active branch ID, state ID, commit ID, topology epoch, thermal epoch, domain, and core state flags.

`timescale 1ns / 1ps

import nvisc_pkg::*;

module nvisc_state_token_if (

input  logic clk,

input  logic rst_n,

input  logic update_i,

input  nvisc_state_token_t token_i,

input  logic branch_advance_i,

input  logic [31:0] next_branch_id_i,

input  logic state_advance_i,

input  logic [31:0] next_state_id_i,

input  logic commit_advance_i,

input  logic [31:0] next_commit_id_i,

input  logic topology_epoch_inc_i,

input  logic thermal_epoch_inc_i,

input  logic set_committed_i,

input  logic set_sealed_i,

input  logic set_rollback_valid_i,

output nvisc_state_token_t token_o

);

nvisc_state_token_t token_q;

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

token_q.branch_id      <= 32'd0;

token_q.state_id       <= 32'd0;

token_q.commit_id      <= 32'd0;

token_q.topology_epoch <= 16'd0;

token_q.thermal_epoch  <= 16'd0;

token_q.domain         <= DOMAIN_BIN;

token_q.valid          <= 1'b1;

token_q.committed      <= 1'b0;

token_q.sealed         <= 1'b0;

token_q.rollback_valid <= 1'b1;

end else begin

if (update_i) begin

token_q <= token_i;

end else begin

if (branch_advance_i) begin

token_q.branch_id <= next_branch_id_i;

token_q.committed <= 1'b0;

token_q.sealed    <= 1'b0;

end

if (state_advance_i) begin

token_q.state_id  <= next_state_id_i;

token_q.committed <= 1'b0;

end

if (commit_advance_i) token_q.commit_id <= next_commit_id_i;

if (topology_epoch_inc_i) token_q.topology_epoch <= token_q.topology_epoch + 16'd1;

if (thermal_epoch_inc_i)  token_q.thermal_epoch  <= token_q.thermal_epoch + 16'd1;

if (set_committed_i) token_q.committed <= 1'b1;

if (set_sealed_i) token_q.sealed <= 1'b1;

if (set_rollback_valid_i) token_q.rollback_valid <= 1'b1;

end

end

end

assign token_o = token_q;

endmodule

# 8. Route MUX Node: nvisc_route_mux_node.sv

Implements the first physical data-selection primitive for local, face, edge, vertex, z-up, and z-down routing inputs.

`timescale 1ns / 1ps

import nvisc_pkg::*;

module nvisc_route_mux_node #(

parameter int DATA_W     = XLEN,

parameter int FACE_PORTS = 12,

parameter int EDGE_PORTS = 24,

parameter int VERT_PORTS = 14

)(

input  logic [DATA_W-1:0] local_i,

input  logic [FACE_PORTS-1:0][DATA_W-1:0] face_i,

input  logic [EDGE_PORTS-1:0][DATA_W-1:0] edge_i,

input  logic [VERT_PORTS-1:0][DATA_W-1:0] vertex_i,

input  logic [DATA_W-1:0] z_up_i,

input  logic [DATA_W-1:0] z_down_i,

input  nvisc_route_src_e src_sel_i,

input  logic [5:0]       port_sel_i,

output logic [DATA_W-1:0] data_o,

output logic              fault_o

);

always_comb begin

data_o  = '0;

fault_o = 1'b0;

unique case (src_sel_i)

ROUTE_SRC_LOCAL: begin

data_o = local_i;

end

ROUTE_SRC_FACE: begin

if (port_sel_i < FACE_PORTS) data_o = face_i[port_sel_i];

else fault_o = 1'b1;

end

ROUTE_SRC_EDGE: begin

if (port_sel_i < EDGE_PORTS) data_o = edge_i[port_sel_i];

else fault_o = 1'b1;

end

ROUTE_SRC_VERTEX: begin

if (port_sel_i < VERT_PORTS) data_o = vertex_i[port_sel_i];

else fault_o = 1'b1;

end

ROUTE_SRC_Z_UP: begin

data_o = z_up_i;

end

ROUTE_SRC_Z_DOWN: begin

data_o = z_down_i;

end

ROUTE_SRC_ZERO: begin

data_o = '0;

end

default: begin

data_o  = '0;

fault_o = 1'b1;

end

endcase

end

endmodule

# 9. Rhombic-Dodeca Router v0: nvisc_rhombic_dodeca_router_v0.sv

Implements a simple route-token issuer for a rhombic-dodecahedron-inspired topology abstraction, including FAST, SAFE, COOL, LOW_POWER, and REDUNDANT policy checks.

`timescale 1ns / 1ps

import nvisc_pkg::*;

module nvisc_rhombic_dodeca_router_v0 #(

parameter int NODE_COUNT = 16

)(

input  logic clk,

input  logic rst_n,

input  logic route_req_i,

input  logic [7:0] source_node_i,

input  logic [7:0] target_node_i,

input  nvisc_route_policy_e policy_i,

input  logic [15:0] topology_epoch_i,

input  logic [15:0] thermal_epoch_i,

input  logic [2:0]  privilege_level_i,

input  logic        aegis_route_approved_i,

input  logic [NODE_COUNT-1:0] node_trusted_i,

input  logic [NODE_COUNT-1:0] node_hot_i,

input  logic [NODE_COUNT-1:0] node_fault_i,

output logic route_valid_o,

output logic route_fault_o,

output logic route_busy_o,

output nvisc_topology_token_t topology_token_o

);

nvisc_topology_token_t token_q;

logic source_in_range;

logic target_in_range;

logic source_ok;

logic target_ok;

logic safe_ok;

logic cool_ok;

logic policy_ok;

logic [15:0] path_id_comb;

assign source_in_range = (source_node_i < NODE_COUNT);

assign target_in_range = (target_node_i < NODE_COUNT);

assign source_ok = source_in_range && !node_fault_i[source_node_i];

assign target_ok = target_in_range && !node_fault_i[target_node_i];

assign safe_ok = node_trusted_i[source_node_i] && node_trusted_i[target_node_i];

assign cool_ok = !node_hot_i[source_node_i] && !node_hot_i[target_node_i];

always_comb begin

policy_ok = 1'b0;

unique case (policy_i)

ROUTE_FAST:      policy_ok = source_ok && target_ok;

ROUTE_SAFE:      policy_ok = source_ok && target_ok && safe_ok && aegis_route_approved_i;

ROUTE_COOL:      policy_ok = source_ok && target_ok && cool_ok;

ROUTE_LOW_POWER: policy_ok = source_ok && target_ok;

ROUTE_REDUNDANT: policy_ok = source_ok && target_ok && safe_ok && cool_ok;

default:         policy_ok = 1'b0;

endcase

end

assign path_id_comb = {source_node_i[5:0], target_node_i[5:0], policy_i[2:0], 1'b0};

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

token_q <= '0;

route_valid_o <= 1'b0;

route_fault_o <= 1'b0;

route_busy_o  <= 1'b0;

end else begin

route_valid_o <= 1'b0;

route_fault_o <= 1'b0;

route_busy_o  <= 1'b0;

if (route_req_i) begin

if (!source_in_range || !target_in_range || !policy_ok) begin

token_q <= '0;

route_fault_o <= 1'b1;

end else begin

token_q.valid            <= 1'b1;

token_q.token_id         <= {8'h52, source_node_i, target_node_i, policy_i};

token_q.source_node      <= source_node_i;

token_q.target_node      <= target_node_i;

token_q.path_id          <= path_id_comb;

token_q.policy           <= policy_i;

token_q.topology_epoch   <= topology_epoch_i;

token_q.thermal_epoch    <= thermal_epoch_i;

token_q.privilege_level  <= privilege_level_i;

token_q.aegis_approved   <= aegis_route_approved_i;

route_valid_o <= 1'b1;

end

end

end

end

assign topology_token_o = token_q;

endmodule

# 10. Phase 0 Core Integration: nexus_v_phase0_core.sv

Connects the register file, ALU slice, state token interface, and debug write path for smoke tests.

`timescale 1ns / 1ps

import nvisc_pkg::*;

module nexus_v_phase0_core (

input  logic clk,

input  logic rst_n,

input  logic          instr_valid_i,

input  nvisc_alu_op_e alu_op_i,

input  nvisc_domain_e domain_i,

input  logic [4:0]    src_a_i,

input  logic [4:0]    src_b_i,

input  logic [4:0]    dst_i,

input  logic          dst_we_i,

input  logic          dbg_r_we_i,

input  logic [4:0]    dbg_r_waddr_i,

input  logic [XLEN-1:0] dbg_r_wdata_i,

input  logic          token_state_advance_i,

input  logic [31:0]   next_state_id_i,

output logic [XLEN-1:0] alu_result_o,

output logic            alu_valid_o,

output logic            zero_o,

output logic            negative_o,

output nvisc_state_token_t state_token_o

);

logic [XLEN-1:0] r_a;

logic [XLEN-1:0] r_b;

logic [XLEN-1:0] alu_result;

logic [MLEN-1:0] m_dummy_a;

logic [MLEN-1:0] m_dummy_b;

logic [MLEN-1:0] m_result;

logic carry;

logic overflow;

assign m_dummy_a = '0;

assign m_dummy_b = '0;

nvisc_regfile u_regfile (

.clk(clk), .rst_n(rst_n),

.r_we((dst_we_i && alu_valid_o) || dbg_r_we_i),

.r_waddr(dbg_r_we_i ? dbg_r_waddr_i : dst_i),

.r_wdata(dbg_r_we_i ? dbg_r_wdata_i : alu_result),

.r_raddr_a(src_a_i), .r_raddr_b(src_b_i),

.r_rdata_a(r_a), .r_rdata_b(r_b),

.f_we(1'b0), .f_waddr('0), .f_wdata('0), .f_raddr_a('0), .f_rdata_a(),

.d_we(1'b0), .d_waddr('0), .d_wdata('0), .d_raddr_a('0), .d_rdata_a(),

.m_we(1'b0), .m_waddr('0), .m_wdata('0), .m_raddr_a('0), .m_rdata_a(),

.t_we(1'b0), .t_waddr('0), .t_wdata('0), .t_raddr_a('0), .t_rdata_a(),

.s_we(1'b0), .s_waddr('0), .s_wdata('0), .s_raddr_a('0), .s_rdata_a(),

.p_we(1'b0), .p_waddr('0), .p_wdata('0), .p_raddr_a('0), .p_rdata_a(),

.c_we(1'b0), .c_waddr('0), .c_wdata('0), .c_raddr_a('0), .c_rdata_a()

);

nvisc_alu_slice u_alu (

.valid_i(instr_valid_i), .op_i(alu_op_i), .domain_i(domain_i),

.a_i(r_a), .b_i(r_b), .m_a_i(m_dummy_a), .m_b_i(m_dummy_b),

.valid_o(alu_valid_o), .result_o(alu_result), .m_result_o(m_result),

.zero_o(zero_o), .negative_o(negative_o), .carry_o(carry), .overflow_o(overflow)

);

nvisc_state_token_if u_state_token (

.clk(clk), .rst_n(rst_n),

.update_i(1'b0), .token_i('0),

.branch_advance_i(1'b0), .next_branch_id_i('0),

.state_advance_i(token_state_advance_i), .next_state_id_i(next_state_id_i),

.commit_advance_i(1'b0), .next_commit_id_i('0),

.topology_epoch_inc_i(1'b0), .thermal_epoch_inc_i(1'b0),

.set_committed_i(1'b0), .set_sealed_i(1'b0), .set_rollback_valid_i(1'b0),

.token_o(state_token_o)

);

assign alu_result_o = alu_result;

endmodule

# 11. Smoke Test Harness: tb_nexus_v_phase0_core.sv

Provides a minimal reset/state-token/ALU ADD/SUB smoke test using the debug register write port.

`timescale 1ns / 1ps

import nvisc_pkg::*;

module tb_nexus_v_phase0_core;

logic clk;

logic rst_n;

logic instr_valid;

nvisc_alu_op_e alu_op;

nvisc_domain_e domain;

logic [4:0] src_a;

logic [4:0] src_b;

logic [4:0] dst;

logic dst_we;

logic dbg_r_we;

logic [4:0] dbg_r_waddr;

logic [XLEN-1:0] dbg_r_wdata;

logic token_state_advance;

logic [31:0] next_state_id;

logic [XLEN-1:0] alu_result;

logic alu_valid;

logic zero;

logic negative;

nvisc_state_token_t state_token;

nexus_v_phase0_core dut (.*,

.instr_valid_i(instr_valid), .alu_op_i(alu_op), .domain_i(domain),

.src_a_i(src_a), .src_b_i(src_b), .dst_i(dst), .dst_we_i(dst_we),

.dbg_r_we_i(dbg_r_we), .dbg_r_waddr_i(dbg_r_waddr), .dbg_r_wdata_i(dbg_r_wdata),

.token_state_advance_i(token_state_advance), .next_state_id_i(next_state_id),

.alu_result_o(alu_result), .alu_valid_o(alu_valid), .zero_o(zero), .negative_o(negative),

.state_token_o(state_token)

);

always #5 clk = ~clk;

task dbg_write(input logic [4:0] addr, input logic [XLEN-1:0] data);

begin

dbg_r_we = 1'b1;

dbg_r_waddr = addr;

dbg_r_wdata = data;

@(posedge clk);

dbg_r_we = 1'b0;

dbg_r_waddr = '0;

dbg_r_wdata = '0;

@(posedge clk);

end

endtask

initial begin

clk = 1'b0;

rst_n = 1'b0;

instr_valid = 1'b0;

alu_op = ALU_NOP;

domain = DOMAIN_BIN;

src_a = '0; src_b = '0; dst = '0; dst_we = 1'b0;

dbg_r_we = 1'b0; dbg_r_waddr = '0; dbg_r_wdata = '0;

token_state_advance = 1'b0; next_state_id = '0;

repeat (4) @(posedge clk);

rst_n = 1'b1;

repeat (2) @(posedge clk);

assert(state_token.valid == 1'b1) else $fatal("State token not valid after reset");

assert(state_token.branch_id == 32'd0) else $fatal("Initial branch_id expected 0");

assert(state_token.state_id == 32'd0) else $fatal("Initial state_id expected 0");

token_state_advance = 1'b1;

next_state_id = 32'd1;

@(posedge clk);

token_state_advance = 1'b0;

@(posedge clk);

assert(state_token.state_id == 32'd1) else $fatal("State token did not advance");

dbg_write(5'd1, 64'd10);

dbg_write(5'd2, 64'd32);

instr_valid = 1'b1;

alu_op = ALU_ADD;

src_a = 5'd1;

src_b = 5'd2;

dst = 5'd3;

dst_we = 1'b1;

@(posedge clk);

@(posedge clk);

assert(alu_valid == 1'b1) else $fatal("ALU valid not asserted");

assert(alu_result == 64'd42) else $fatal("ALU ADD expected 42, got %0d", alu_result);

alu_op = ALU_SUB;

src_a = 5'd2;

src_b = 5'd1;

dst = 5'd4;

@(posedge clk);

@(posedge clk);

assert(alu_result == 64'd22) else $fatal("ALU SUB expected 22, got %0d", alu_result);

$display("Nexus-V Phase 0 smoke tests passed.");

$finish;

end

endmodule

# 12. Phase 0 Smoke Tests

| Test | Expected result |
| --- | --- |
| Reset state token | valid=1, branch_id=0, state_id=0, rollback_valid=1 |
| Advance state token | state_id updates to requested next_state_id |
| r0 zero behavior | writes to r0 do not change r0 output |
| Debug preload | debug port can write r1 and r2 for deterministic ALU tests |
| ALU ADD | 10 + 32 = 42 |
| ALU SUB | 32 - 10 = 22 |
| Bitwise ops | AND/OR/XOR/NOT produce expected masks |
| MVL ternary add | MADD wraps modulo 3 |
| MVL quaternary add | MADD wraps modulo 4 |
| Route MUX node | Selected local/face/edge/vertex/Z port appears at output or faults on invalid index |
| Router v0 FAST | Valid route token when source/target are in range and not faulty |
| Router v0 SAFE | Requires trusted endpoints and Aegis route approval |
| Router v0 COOL | Rejects hot endpoints |

# 13. Route Fabric Development Path

The route MUX node and rhombic-dodeca router v0 are deliberately simple. They establish the contract: route inputs are selectable, route policies are checked, and a successful route emits a topology token. Later phases can replace table-derived path IDs with hardware pathfinding, reservation tables, congestion maps, and real face/edge/vertex adjacency tables.

| Phase | Route hardware upgrade |
| --- | --- |
| 0 | Route MUX node and router v0 token issuer |
| 1 | Route reservation table and topology epoch invalidation |
| 2 | Adjacency-table path selection for face/edge/vertex/layer routing |
| 3 | Thermal-aware route scoring and route handle expiry |
| 4 | Redundant route duplication and compare logic |
| 5 | Integration with Aegis route audit and RSM state-token routing |

# 14. Known Phase 0 Limitations

The router v0 does not yet compute true paths across all rhombic-dodecahedron faces, edges, and vertices; it issues a deterministic path_id placeholder.

The topology token is functional metadata, not yet backed by a reservation SRAM or route occupancy table.

Tensor and state registers are represented as handles, not full hardware tensor/state stores.

Floating, decimal, and capability registers exist in the register file but are not yet driven by execution units.

Aegis route approval is represented as an input signal; the real Aegis Enclave is introduced in later RTL phases.

The ALU slice is scalar; multiple slices and volumetric interconnect are later extensions.

# 15. Immediate Next Implementation Tasks

| Priority | Task | Output |
| --- | --- | --- |
| P0 | Compile package, regfile, ALU slice, state token interface, and core | Clean compile under simulator |
| P0 | Run tb_nexus_v_phase0_core | Smoke tests pass |
| P1 | Add tb_nvisc_route_mux_node | MUX selection and invalid-index faults verified |
| P1 | Add tb_nvisc_rhombic_dodeca_router_v0 | FAST/SAFE/COOL policies verified |
| P2 | Add route_trace output in testbench | JSON or text trace matching NVVM route trace fields |
| P2 | Add state-token epoch invalidation test | Route token validity changes when topology/thermal epoch changes |