Nexus-V RTL Microarchitecture Specification

v0.1 - Volumetric ALU, Aegis Enclave, and Repository-State Memory Controller

Standalone hardware-definition document for the NVISC/Nexus-V architecture

# 0. Purpose and Scope

This document translates the NVISC/Nexus-V software-visible architecture into a first RTL microarchitecture target. It removes the artificial choice between beginning with only the Volumetric ALU, only the Aegis Enclave, or only the Repository-State Memory Controller. The correct hardware path is to define all three as mutually dependent blocks, then implement a minimal synchronized subset first.

The purpose is not to claim that the first implementation physically constructs a literal rhombic-dodecahedral chip. The v0.1 hardware target is a synthesizable RTL model with topology-aware routing abstractions, token tables, commit gates, and state-versioned memory semantics. The physical 3D/polyhedral substrate can be approached later once the functional guarantees are proven.

## 0.1 Hardware Guarantees

State can branch, checkpoint, commit, roll back, and merge through explicit metadata tables.

Routes are explicit, tokenized, policy-aware, and auditable.

Committed architectural state is gated by Aegis authorization.

Copy-on-write and rollback are implemented through metadata and pointer rebinding rather than full memory copying.

The Volumetric ALU can execute ordinary ALU operations while interacting with a topology route fabric.

ATTEST and MINREP are issued by isolated hardware units, not ordinary user code.

The first RTL target uses encoded binary representations for MVL/ternary/quaternary domains before attempting physical multi-level signaling.

# 1. Hardware Requirements from the NVISC Plan

| Requirement | Hardware interpretation |
| --- | --- |
| 3D dense state-space processor | Represent the compute fabric as nodes, layers, faces, edges, vertices, and sectors with a route MUX network and topology descriptor tables. |
| Parallel-in-depth execution | Expose z/layer state movement, branch channels, merge fabric, and checkpoint lattice metadata even if implemented as tables in v0.1. |
| Git-like states | Implement Branch Table, State Table, Commit Table, Snapshot Table, Delta Log, and Rollback Map. |
| More-than-binary logic | Use domain tags and encoded binary MVL registers first; later physical ternary/quaternary signaling remains optional. |
| Branch explosion control | Expose budget counters, branch TTL, state-token quotas, and RSM garbage-collection support. |
| Formal merge semantics | Add VPHI/Merge Resolver hardware hooks and branch score fields. |
| Aegis safeguard auditing | Use isolated policy store, audit FSM, token issuer, and commit gate. |
| Anti-cheat / privacy proof | Generate local claims and minimal reports through hardware redaction and quote units. |
| Thermal/error constraints | Expose thermal epoch, node health, ECC status, route masking, and throttle/migrate signals. |
| Alternative topologies | Parameterize topology descriptors for rhombic dodecahedron, dodecahedron, octahedron, tetrahedron, triangular bipyramid, and isohedral profiles. |

# 2. Top-Level Nexus-V RTL Block Diagram

Nexus-V Core v0.1
├─ Front End
│  ├─ Instruction Fetch
│  ├─ Decode
│  └─ Micro-op Expander
├─ Register and Token Cluster
│  ├─ Integer / FP / Decimal Registers
│  ├─ MVL Domain Registers
│  ├─ State Registers
│  ├─ Predicate Registers
│  └─ Capability Registers
├─ Volumetric ALU
│  ├─ ALU Slice Array
│  ├─ MVL Encoded Logic Unit
│  ├─ FP/Decimal Numeric Unit
│  ├─ Local Route Ports
│  └─ 3D Route MUX Fabric
├─ Topology Router
│  ├─ Face / Edge / Vertex / Layer Decoders
│  ├─ Route Table / Path Lookup
│  ├─ Path Reservation Table
│  └─ Route Token Table
├─ Repository-State Memory Controller
│  ├─ Branch Table
│  ├─ State Table
│  ├─ Commit Table
│  ├─ Page Version Table
│  ├─ COW Bitmap
│  ├─ Delta Log Buffer
│  └─ Rollback Engine
├─ Aegis Enclave
│  ├─ Policy Store
│  ├─ Audit FSM
│  ├─ Commit Gate
│  ├─ Token Issuer
│  ├─ Hash/MAC/Quote Unit
│  └─ MINREP Redaction Unit
├─ Thermal / Reliability Monitor
├─ Commit / Retire Unit
└─ Trace / RTL Test-Vector Export

# 3. RTL v0.1 Design Boundary

Nexus-V v0.1 should implement the smallest hardware subset that proves the architecture. Anything not essential to proving branchable state, auditable routes, Aegis-gated commits, and rollback should be represented by stubs or table-driven models.

| Implement in v0.1 | Defer until later |
| --- | --- |
| B.ADD, B.SUB, B.LOAD, B.STORE, B.CMP, B.HALT | Out-of-order execution, branch prediction, superscalar issue |
| S.SNAP, S.BRANCH, S.COMMIT, S.ROLLBACK | Full speculative merge fabric |
| RSM.COW metadata and minimal RSM.DELTA | High-performance state compression engine |
| Table-driven G.ROUTE / G.GEO_PATH | Hardware pathfinding over arbitrary physical topology |
| A.AUDIT, A.ATTEST placeholder, P.MINREP placeholder | Full production cryptographic certification stack |
| Route token table, audit token table, state table | Physical multi-level ternary/quaternary circuits |
| Thermal/status registers and route masks | Accurate semiconductor thermal solver |

# 4. Clock, Reset, and Pipeline Model

## 4.1 Clock Domains

| Clock domain | Purpose | v0.1 recommendation |
| --- | --- | --- |
| core_clk | Instruction pipeline, register file, ALU issue/retire. | Use as main clock. |
| rsm_clk | RSM metadata tables, COW map, delta log, rollback engine. | May be tied to core_clk in v0.1. |
| aegis_clk | Policy store, audit FSM, hash/MAC/quote, commit gate. | May be tied to core_clk in v0.1; isolate reset path. |
| route_clk | Topology route table, route MUX control, token table. | May be tied to core_clk in v0.1. |
| tensor_clk | Future tensor/NPU fabric. | Stub in v0.1. |
| io_clk | I/O provenance and attestation inputs. | Stub/status registers in v0.1. |
| debug_clk | Trace export and test-vector capture. | Optional synchronous trace FIFO. |

## 4.2 Minimal Pipeline

IF   Instruction Fetch
ID   Decode / Micro-op expansion
REN  Register and state-token rename
ISS  Issue to ALU, RSM, Route, or Aegis request queue
EX   Execute local operation or begin external unit request
MEM  Memory / RSM metadata access
AG   Aegis Gate check for policy-sensitive state transitions
RET  Retire / commit architectural state token

The AG stage is the distinctive Nexus-V stage. Ordinary CPUs retire instructions after execution and exception checks. Nexus-V retires policy-sensitive state only after execution, metadata consistency checks, and Aegis authorization.

# 5. Volumetric ALU

## 5.1 Purpose

The Volumetric ALU executes ordinary arithmetic and logic while exposing the physical/topological movement model required by NVISC. In v0.1, volumetric behavior is represented as ALU slices connected through a topology-aware route MUX fabric. Later versions may bind those slices to physical 3D or chiplet geometry.

## 5.2 ALU Slice

ALU_SLICE {
    slice_id;
    local_reg_a;
    local_reg_b;
    route_in;
    opcode;
    domain;
    result;
    flags;
    route_out;
    state_token_in;
    state_token_out;
}

| Signal | Direction | Meaning |
| --- | --- | --- |
| alu_req_valid | in | Instruction issue request for this slice. |
| alu_req_ready | out | Slice can accept request. |
| alu_opcode | in | ALU/MVL operation code. |
| alu_domain | in | BIN, TERN, QUAD, NSTATE, DEC, FP, etc. |
| alu_src_a / alu_src_b | in | Local or routed operands. |
| alu_result | out | Operation result. |
| alu_flags | out | Zero/carry/overflow/domain-error flags. |
| state_in / state_out | in/out | State token sequencing for mutation tracking. |

## 5.3 Encoded MVL First

The v0.1 RTL should not attempt physical ternary or quaternary voltage signaling. It should encode multi-valued states in binary containers with domain metadata. This avoids manufacturing and noise-margin risk while preserving ISA semantics.

MVL_VALUE {
    encoded_value;       // e.g. 0, 1, 2 for TERN
    domain_id;           // BIN, TERN, QUAD, NSTATE
    max_state;           // N-1 for NSTATE
    confidence;
    error_score;
}

# 6. Rhombic-Dodecahedron Route Fabric

## 6.1 Conceptual Geometry

The rhombic dodecahedron profile is treated as a routing topology profile. Its faces, edges, and vertices become logical routing ports and adjacency groups. This gives the RTL a clean abstraction for physical 3D routing without hardcoding a final silicon layout.

| Geometry element | RTL interpretation |
| --- | --- |
| 12 faces | Broad directional face ports or face-region routing groups. |
| 24 edges | Inter-face transfer channels. |
| 14 vertices | High-connectivity junctions or aggregation points. |
| Z/depth ports | Layer-to-layer transformation/history movement. |
| Shell port | Aegis boundary / outer audit interface. |
| Core-volume port | Internal compute-volume access path. |

## 6.2 Parameterized Route Ports

parameter FACE_PORTS = 12;
parameter EDGE_PORTS = 24;
parameter VERT_PORTS = 14;
parameter Z_PORTS    = 2;

route_mux_inputs = {
    local_result,
    face_in[0:FACE_PORTS-1],
    edge_in[0:EDGE_PORTS-1],
    vertex_in[0:VERT_PORTS-1],
    z_up_in,
    z_down_in,
    shell_in,
    corevol_in
};

route_out = route_mux_select(route_mux_inputs, route_sel);

## 6.3 Route Tables

v0.1 route selection should be table-driven. Hardware pathfinding can be added later. The route table maps a source node, target node, and route policy into a path ID and MUX select sequence.

ROUTE_TABLE[source_node][target_node][policy] -> ROUTE_ENTRY

ROUTE_ENTRY {
    valid;
    path_id;
    hop_count;
    mux_select_sequence_ptr;
    latency_cost;
    thermal_cost;
    energy_cost;
    trust_mask_required;
}

## 6.4 Route Token Table

ROUTE_TOKEN_ENTRY {
    valid;
    token_id;
    source_node;
    target_node;
    path_id;
    route_policy;
    topology_epoch;
    thermal_epoch;
    privilege_level;
    aegis_approved;
    reserved;
}

## 6.5 G.ROUTE Cycle Sequence

| Cycle | Action |
| --- | --- |
| 0 | Decode G.ROUTE or G.GEO_PATH and enqueue route request. |
| 1 | Resolve source/target labels to node IDs. |
| 2 | Read topology descriptor and route table. |
| 3 | Check route policy, node health, trust mask, and thermal mask. |
| 4 | Reserve path and generate MUX select sequence. |
| 5 | Issue route token and update route token table. |
| 6+ | Move data/state through selected route hops or mark ready for execution fabric. |

# 7. Aegis Enclave

## 7.1 Purpose

The Aegis Enclave is the hardware trust root for Nexus-V. It does not merely check payloads. It checks whether safeguards remain active, whether state transitions are allowed, and whether minimal proof can be issued without exposing private data.

## 7.2 Isolation Boundary

| Boundary rule | Hardware implication |
| --- | --- |
| Policy store cannot be directly written by user code. | Writes require Aegis privilege or signed update sequence. |
| Audit tokens cannot be forged by predicates. | Only Token Issuer can allocate valid audit token IDs. |
| ATTEST/MINREP cannot expose raw private data by default. | Report builder uses claim filters and redaction masks. |
| Aegis reset path is protected. | Independent reset/lockdown latch and monotonic counter. |
| Safeguard tamper is high-risk. | Tamper event latch can force refuse/quarantine/lockdown response. |

## 7.3 Aegis Hardware Blocks

Aegis Enclave
├─ Aegis Control FSM
├─ Policy Store ROM/RAM
├─ Capability Table
├─ Audit Token Table
├─ Commit Gate
├─ Hash Engine
├─ MAC / Quote Engine
├─ RNG / Nonce Unit
├─ Minimal Report Builder
├─ Redaction Filter
├─ Secure Counter / Monotonic Timer
├─ Tamper Event Latch
└─ Aegis Debug Lock

## 7.4 Audit Token Entry

AUDIT_TOKEN_ENTRY {
    valid;
    token_id;
    subject_state_id;
    subject_hash;
    policy_id;
    valid_for_mask;      // COMMIT, SEAL, ATTEST, MINREP, etc.
    privilege_level;
    topology_epoch;
    thermal_epoch;
    issued_counter;
    expires_on_state_change;
}

## 7.5 Commit Gate Logic

commit_allow =
    audit_token.valid
 && audit_token.subject_hash == candidate_state_hash
 && policy_allows_commit
 && privilege_ok
 && rollback_available
 && numeric_integrity_ok
 && topology_token_valid
 && thermal_safe
 && !tamper_latch;

if (!commit_allow) {
    if (tamper_latch) action = LOCKDOWN_OR_QUARANTINE;
    else if (!policy_allows_commit) action = REFUSE;
    else action = DENY_OR_ROLLBACK;
}

## 7.6 ATTEST Path

1.  Receive ATTEST request from decode or Aegis queue.

2.  Collect permitted measurements: code hash, state hash, policy hash, topology epoch, RSM integrity, route token status, Aegis status.

3.  Bind measurements to nonce and monotonic counter.

4.  Hash the measurement bundle.

5.  Sign or MAC the hash using the Aegis quote unit.

6.  Return a capability/proof token.

## 7.7 MINREP Path

1.  Receive MINREP request and proof source handle.

2.  Load policy-defined claim allowlist.

3.  Select only minimal permitted claims.

4.  Apply redaction mask to private fields.

5.  Hash and sign/MAC the minimal report.

6.  Return MINREP token without exposing raw memory, unrelated processes, or private files.

# 8. Repository-State Memory Controller

## 8.1 Purpose

The RSM Controller implements hardware-level support for snapshots, branch creation, copy-on-write, deltas, rollback, state sealing, and commit metadata. It is the physical mechanism that makes Git-like state execution possible.

## 8.2 RSM Hardware Blocks

RSM Controller
├─ State ID Allocator
├─ Branch Table
├─ Commit Table
├─ Snapshot Table
├─ Page Version Table
├─ Copy-On-Write Bitmap
├─ Delta Log Buffer
├─ Rollback Map
├─ Seal Table
├─ Reference Counter
├─ RSM TLB
├─ RSM Cache
├─ RSM Garbage Collector
└─ Memory Hash/ECC Unit

## 8.3 Table Records

BRANCH_ENTRY {
    valid;
    branch_id;
    parent_branch_id;
    head_state_id;
    root_state_id;
    status;
    score;
    budget_id;
}

STATE_ENTRY {
    valid;
    state_id;
    branch_id;
    parent_state_id;
    page_table_root;
    delta_log_ptr;
    state_hash;
    sealed;
    committed;
    rollback_valid;
    audit_status;
}

PAGE_VERSION_ENTRY {
    virtual_page;
    physical_page;
    owner_state_id;
    cow;
    dirty;
    refcount;
    sealed;
    ecc_status;
}

## 8.4 Copy-On-Write Hardware

When a branch is created, pages are not copied. Parent pages are shared, refcounts increase, and COW bits are set. A private physical copy is made only when a child writes to a shared page.

| Step | COW action |
| --- | --- |
| Branch create | Share parent page mappings; increment refcounts; mark COW. |
| Child write to shared page | Trap/stall to RSM Controller. |
| Private page allocation | Allocate new physical page or cache-line copy region. |
| Mapping update | Update child page version table entry. |
| Delta append | Append write intent and page mapping change to delta log. |
| Resume | Complete original store into child-owned page. |

## 8.5 COW Write Pipeline

| Cycle | Action |
| --- | --- |
| 0 | Store instruction issues. |
| 1 | RSM TLB lookup. |
| 2 | Detect COW page. |
| 3 | Stall pipeline and request copy. |
| 4-N | Copy page or cache line. |
| N+1 | Update Page Version Table. |
| N+2 | Append Delta Log entry. |
| N+3 | Complete store. |

## 8.6 Instant Rollback Hardware

Instant rollback should mean instant pointer rollback. The controller changes the active state root pointer and invalidates descendant-only tokens instead of copying old data back over new data.

rollback = {
    active_state_id       = target_state_id;
    active_page_root      = STATE_TABLE[target_state_id].page_table_root;
    active_branch_head    = target_state_id;
    invalidate_descendant_tokens();
    flush_speculative_pipeline();
}

| Cycle | Rollback action |
| --- | --- |
| 0 | Decode S.ROLLBACK target_state. |
| 1 | Check target state exists. |
| 2 | Check rollback_valid/sealed status. |
| 3 | Flush speculative pipeline. |
| 4 | Rebind active page-table root and state ID. |
| 5 | Invalidate descendant audit, route, tensor, quantum, and branch-private tokens. |
| 6 | Resume from rollback PC/state. |

## 8.7 Commit Hardware

| Cycle | Commit action |
| --- | --- |
| 0 | Decode S.COMMIT candidate. |
| 1 | Lookup or compute candidate state hash. |
| 2 | Send candidate to Aegis Commit Gate. |
| 3 | If allowed, seal pages and mark committed. |
| 4 | Update Commit Table and Branch Table head. |
| 5 | Emit commit token and retire instruction. |

# 9. Hardware Interfaces

| Interface | Producer | Consumer | Purpose |
| --- | --- | --- | --- |
| route_req | Core/Decode | Topology Router | Request route or GEO_PATH token. |
| route_resp | Topology Router | Core/Route Token Table | Return route result, fault, or token ID. |
| rsm_req | Core/MEM stage | RSM Controller | Load/store/snapshot/branch/rollback metadata operations. |
| rsm_resp | RSM Controller | Core/Commit Unit | Return state token, memory token, fault, or rollback result. |
| aegis_req | Core/AG stage | Aegis Enclave | Audit, attest, minrep, commit-gate request. |
| aegis_resp | Aegis Enclave | Core/Commit Unit | Allow/deny/refuse/quarantine, token ID, proof ID. |
| thermal_status | Thermal Monitor | Router/Aegis/NVRT hooks | Route masks, hotspot, throttle, migrate. |
| trace_event | All units | Trace Export | Cycle-level debug and RTL test-vector capture. |

# 10. Instruction-to-Hardware Mapping

| Instruction | Hardware path |
| --- | --- |
| B.ADD / B.SUB | Decode -> ALU Slice -> State token update -> Retire. |
| B.LOAD / B.STORE | Decode -> RSM TLB/Cache -> COW check if store -> Retire. |
| S.SNAP | Decode -> RSM Snapshot Table -> State token emit. |
| S.BRANCH | Decode -> Branch Table allocation -> Page refcount/COW setup -> Branch token emit. |
| S.ROLLBACK | Decode -> RSM Rollback Engine -> Pipeline flush -> Root pointer rebind -> Token invalidation. |
| S.COMMIT | Decode -> Candidate state hash -> Aegis Commit Gate -> Commit Table update. |
| G.ROUTE / G.GEO_PATH | Decode -> Topology Router -> Route Table -> Path Reservation -> Route Token Table. |
| A.AUDIT | Decode -> Aegis Audit FSM -> Audit Token Table. |
| A.ATTEST | Decode -> Measurement Hash -> Quote/MAC -> Proof token. |
| P.MINREP | Decode -> Claim filter -> Redaction -> Hash/MAC -> Minimal-report token. |
| RSM.COW | Decode -> COW Bitmap/Page Version Table updates. |
| RSM.DELTA | Decode -> Delta Log Buffer entry. |

# 11. Token Epochs and Invalidation

Token invalidation is the hardware mechanism that prevents stale future-state artifacts from corrupting later execution. Each token class should carry an epoch or root-state binding.

| Token class | Invalidated by |
| --- | --- |
| Audit token | Subject state mutation, policy epoch change, privilege context change, rollback of subject. |
| Route token | Topology epoch change, thermal epoch change, route release, node fault, privilege change. |
| Memory token | Rollback, RSM GC, page remap, seal violation, ECC failure. |
| Branch token | Prune, rollback, merge collapse, branch TTL expiry, budget violation. |
| Quantum token | Measurement/collapse, QSYNC failure, rollback, qmode context change. |
| Proof/MINREP token | Policy change, measurement expiry, nonce/counter invalidity, redaction failure. |

# 12. Fault and Response Model

| Fault | Detection point | Default response |
| --- | --- | --- |
| ROUTE_POLICY_DENIED | Topology Router/Aegis route check | Deny route, raise route fault, preserve state. |
| TOPOLOGY_ROUTE_FAILED | Route Table / Path Reservation | Fault or fallback route if policy permits. |
| AUDIT_REQUIRED | Commit Gate | Block commit and request audit. |
| COMMIT_SUBJECT_MISMATCH | Aegis Commit Gate | Refuse commit; mark candidate suspicious. |
| COW_ALLOCATION_FAILED | RSM Controller | Stall, request GC, then fault if unresolved. |
| ROLLBACK_TARGET_INVALID | RSM Rollback Engine | Refuse rollback and quarantine branch if necessary. |
| THERMAL_UNSAFE | Thermal Monitor / Commit Gate | Throttle, migrate, or refuse high-risk commit. |
| SAFEGUARD_TAMPER | Aegis Tamper Latch | Lockdown, quarantine, or recovery executor path. |

# 13. RTL Test Vectors

NVVM/NVRT traces should generate deterministic RTL test vectors. Every vector should define initial tables, instruction sequence, expected token changes, and final state hash.

| Vector file | Purpose |
| --- | --- |
| route_test_vectors.json | Validate G.ROUTE/G.GEO_PATH table lookup, route tokens, and policy masks. |
| aegis_commit_gate_vectors.json | Validate audit-token subject matching and commit allow/deny/refuse. |
| rsm_cow_vectors.json | Validate COW page detection, page copy, refcount, and delta log updates. |
| rollback_vectors.json | Validate pointer rollback, pipeline flush, and token invalidation. |
| attest_minrep_vectors.json | Validate measurement bundle, redaction, and proof-token issuance. |
| branch_budget_vectors.json | Validate branch table budget fields and prune/collapse hooks. |
| thermal_route_vectors.json | Validate ROUTE_COOL and thermal epoch invalidation. |

# 14. Engineering Risks and Mitigations

| Risk | Why it matters | Mitigation |
| --- | --- | --- |
| State metadata explosion | Branch/state tables may grow faster than compute. | Enforce branch budgets, GC, COW, deltas, and table size limits. |
| COW latency | First write after branch can stall. | Use cache-line COW in v0.1 and pre-copy hints later. |
| Aegis bottleneck | Commit and attestation may serialize progress. | Cache audit tokens, batch audits, and separate Aegis queue. |
| Route fabric complexity | Full polyhedral routing is large. | Start with table-driven routes and parameterized MUX ports. |
| Rollback correctness | Stale tokens can corrupt state. | Hardware epoch invalidation and token subject binding. |
| Crypto cost | Full signature hardware is area-expensive. | Use hash/MAC placeholder in v0.1; later add production-grade quote unit. |
| Physical MVL difficulty | Ternary/quaternary voltage levels are noisy. | Use encoded binary MVL first. |
| Thermal mismatch | Simulator may not match physical chip. | Keep route policies abstract; validate with status registers first. |

# 15. Implementation Roadmap

| Phase | Hardware target |
| --- | --- |
| Phase 0 | RTL skeleton, register file, ALU slice, state token register, halt/add/load/store smoke tests. |
| Phase 1 | RSM tables, SNAP, BRANCH, COMMIT metadata, COW bitmap, basic rollback pointer swap. |
| Phase 2 | Aegis audit token table, commit gate, placeholder ATTEST/MINREP token issuance. |
| Phase 3 | Topology route table, route token table, G.ROUTE/G.GEO_PATH, ROUTE_SAFE/ROUTE_COOL masks. |
| Phase 4 | Trace export and NVVM/NVRT-generated test vectors. |
| Phase 5 | Thermal/reliability status registers, ECC hooks, route invalidation epochs. |
| Phase 6 | Optional tensor interface and MVL expanded domain behavior. |

# 16. Final Hardware Definition Rule

Nexus-V hardware is defined by three physical guarantees:

1. State can branch and roll back through RSM metadata.
2. Routes are explicit, tokenized, and auditable through the topology fabric.
3. Commits require trusted authorization through the Aegis Commit Gate.

The Volumetric ALU, Aegis Enclave, and RSM Controller are not separate optional tracks.
They are the minimum hardware triangle required for NVISC to function.

End of Nexus-V RTL Microarchitecture Specification v0.1