Nexus-V Core Completeness Specification v0.2

Conventional CPU Core Completion, SMT Roadmap, Cache/Bus/Memory Hierarchy, and Octa-Core Scalability Plan

| Field | Value |
| --- | --- |
| Document ID | NV-CORE-COMPLETE-v0.2 |
| Scope | Single-core CPU completion specification with explicit roadmap hooks for SMT, high IPC, shared cache fabric, custom buses, memory controllers, and future octa-core scaling. |
| Baseline | Nexus-V v0.1 SoC bring-up has already reached Verilator lint, C++ model generation, executable link, and 100-cycle reset smoke execution. |
| Implementation status | This document completes the v0.2 design/specification checklist. It does not claim all RTL modules are physically implemented yet. |
| Target direction | Single-core first, SMT-ready second, cache/bus/memory-complete third, octa-core later. |

# 1. Executive Definition

Nexus-V Core Completeness Specification v0.2 defines the conventional CPU-core layer missing from the earlier Nexus-V architecture work. The prior system already has the unusual Nexus-V domains: RSM versioned state, Aegis commit/audit, topology routing, tensor/NPU execution, and QSIM/QHW quantum bridging. v0.2 adds the ordinary processor structures needed to make Nexus-V a real CPU core rather than only a SoC control shell around specialized accelerators.

v0.1 proved: SoC top-level can elaborate, compile, link, and reset-run in simulation.
v0.2 defines: instruction fetch/decode/control, PC/branch, LSU, MMU/TLB, caches, CSR, interrupts, bus/memory interfaces, vector/SIMD, and future SMT/multicore hooks.

# 2. Scope Boundary and Honesty Clause

This document marks requirements as [x] because the v0.2 specification now defines them architecturally.

The [x] marks do not mean every block has already passed RTL simulation or synthesis.

The immediate implementation target remains one Nexus-V core with one architectural hardware thread.

SMT, high-IPC out-of-order execution, L3 cache, and octa-core operation are included as roadmap-compatible architecture requirements rather than instant implementation requirements.

All new CPU blocks must preserve Nexus-V identity tokens: core_id, thread_id, branch_id, state_id, commit_id, privilege, Aegis policy tag, and RSM memory-root identity.

# 3. v0.2 Single-Core Target Configuration

| Component | v0.2 target | Future scaling |
| --- | --- | --- |
| CPU cores | 1 Nexus-V core | 8 cores in octa-core v0.5 |
| Hardware threads | 1 active hardware thread, SMT-ready metadata fields | 2-way SMT, then optional 4-way SMT |
| Fetch/decode | In-order baseline with clean control interfaces | High-IPC fetch queue, uop cache, branch predictor |
| Execution | Scalar ALU + branch + LSU + optional vector/SIMD + dispatch paths | Multi-lane/out-of-order execution cluster |
| Memory | L1I/L1D + optional L2 interface + RSM-aware memory protocol | Shared L2/L3, coherence, multicore ownership |
| System services | Shared RSM, Aegis, topology, tensor, quantum services | Shared fabric with per-core IDs and Aegis channels |

# 4. Architectural Principle

Every instruction, memory operation, cache line, bus transaction, thread context, and speculative result must carry enough identity to answer:
who executed it, under which thread, on which core, in which branch/state timeline, under which Aegis policy, and whether it can safely commit.

# 5. Core Block Diagram

Nexus-V Core v0.2
├─ Front-End
│  ├─ Program Counter / next-PC logic
│  ├─ Instruction Fetch Unit
│  ├─ L1 Instruction Cache
│  ├─ Branch Unit / branch predictor hook
│  └─ Instruction Decoder / Control Unit
├─ Execution Core
│  ├─ Scalar Register File
│  ├─ Control/Status Registers
│  ├─ Integer ALU Cluster
│  ├─ LSU / Load-Store Queue baseline
│  ├─ Optional Vector Register File
│  ├─ Optional Vector ALU / SIMD Unit
│  ├─ Tensor Dispatch Path
│  ├─ Route Dispatch Path
│  ├─ Quantum Dispatch Path
│  └─ Commit/Audit Path
├─ Memory Subsystem
│  ├─ MMU / TLB
│  ├─ L1 Data Cache
│  ├─ Optional L2 Interface
│  ├─ NexusBus Master Interface
│  └─ RSM-aware cache tags
├─ Thread/State Subsystem
│  ├─ Thread Context Table placeholder
│  ├─ Per-thread PC/state-token/branch-token placeholders
│  ├─ Token Scoreboard
│  └─ RSM-safe retirement hooks
└─ System Interfaces
   ├─ Interrupt Controller input path
   ├─ Timer input path
   ├─ Aegis policy/attestation channel
   ├─ Reset/clock-domain boundary
   └─ Debug/trace channel

# 6. Requirement Completion Matrix

| Status | Requirement | v0.2 definition |
| --- | --- | --- |
| [x] | Instruction fetch unit | Defines PC-driven instruction request generation, L1I access, fetch-valid/fetch-ready handshakes, branch redirect handling, and future fetch queue interface. |
| [x] | Instruction decoder | Defines NVISC instruction classification into ALU, branch, load/store, CSR, vector, tensor, route, quantum, RSM, and Aegis micro-operations. |
| [x] | Program counter / branch unit | Defines next-PC selection, branch target calculation, trap-vector redirection, rollback redirect, and future branch prediction hooks. |
| [x] | Control/status registers | Defines CSR bank for privilege, trap cause, interrupt enable, core/thread identity, state-token views, and Aegis/RSM status mirrors. |
| [x] | LSU | Defines address generation, alignment checks, load/store valid-ready, memory fault reporting, store buffering, and RSM/COW interaction hooks. |
| [x] | MMU / TLB | Defines virtual-to-physical translation, ASID/process context, page permissions, Aegis region permissions, and RSM page-root binding. |
| [x] | L1 instruction cache | Defines private per-core instruction cache with invalidation on code updates, privilege changes, and Aegis code quarantine. |
| [x] | L1 data cache | Defines private per-core data cache with RSM-aware tags and rollback/commit/seal interaction. |
| [x] | L2 cache | Defines optional v0.2 interface and v0.4 shared/private L2 implementation path. |
| [x] | Memory controller / AXI-like bus | Defines NexusBus/AXI-like valid-ready transaction model with token metadata and memory response semantics. |
| [x] | Interrupt controller | Defines timer/external/software interrupt inputs, priority, privilege handoff, and per-thread interrupt state. |
| [x] | Timer | Defines cycle counter, compare interrupt, scheduling tick, and trace timestamp source. |
| [x] | PLL / reset / clock-domain controller | Defines external clock assumption for simulation and future PLL/reset/CDC boundary requirements for FPGA/ASIC. |
| [x] | Conventional vector register file | Defines optional SIMD vector architectural registers separate from tensor descriptors. |
| [x] | Vector ALU / SIMD execution unit | Defines optional lane-based SIMD integer/floating/vector operation unit. |
| [x] | Multiple Nexus-V core instances | Defines future replication contract, not v0.2 physical replication. |
| [x] | Per-core program counters | Defines one PC per future core and per-thread PC arrays for SMT. |
| [x] | Per-core register files | Defines private architectural/physical register files per future core. |
| [x] | Per-core state tokens | Defines state-token ownership per core/thread, synchronized through RSM. |
| [x] | Shared RSM coherence model | Defines shared RSM as version authority with ownership, seals, rollback invalidation, and COW coherence. |
| [x] | Inter-core interrupt/signaling system | Defines future IPI model with core_id/thread_id routing. |
| [x] | Shared L2/cache fabric | Defines future shared cache fabric and coherence hooks. |
| [x] | Memory consistency model | Defines ordered baseline with future relaxed/acquire-release/atomic extensions. |
| [x] | Branch/state ownership rules between cores | Defines ownership transfer, shared read, speculative fork, commit, and rollback rules. |
| [x] | Aegis per-core attestation IDs | Defines core_id/thread_id in audit/attest/minrep tokens. |
| [x] | NVRT hardware/software scheduler for threads | Defines scheduler contract for OS/NVRT to manage hardware threads, tensor jobs, and branch budgets. |

# 7. Front-End Specification

## 7.1 Instruction Fetch Unit

Consumes current PC and produces instruction fetch requests.

Supports fetch_valid/fetch_ready request and response handshake.

Fetches from L1I first, then from the memory/bus interface on miss.

Accepts redirect from branch unit, trap unit, interrupt unit, and RSM rollback recovery.

Carries core_id, thread_id, privilege, branch_id, and state_id in fetch metadata.

## 7.2 Instruction Decoder / Control Unit

Classifies NVISC instructions into execution domains.

Emits micro-operation descriptors for scalar, memory, branch, CSR, vector, tensor, route, quantum, RSM, and Aegis paths.

Performs operand decode, immediate extraction, privilege classification, and illegal-instruction detection.

Generates issue eligibility metadata for the future dynamic sequence unit.

## 7.3 Program Counter / Branch Unit

Tracks current PC for the active thread.

Computes PC+instruction_length, branch targets, call/return targets, trap vectors, interrupt vectors, and rollback recovery vectors.

Supports branch prediction hooks but may begin with static not-taken baseline.

Flushes or redirects fetch/decode on branch, trap, interrupt, or rollback events.

# 8. Execution and Register-State Specification

## 8.1 Scalar Register File Cleanup

Defines integer register read/write ports, debug write path, architectural zero register behavior if used, and future physical register mapping.

Each register write is tagged with thread_id, branch_id, state_id, and retire permission.

Future out-of-order mode maps architectural registers through rename tables into a physical register file.

## 8.2 Integer ALU Cluster

Baseline single ALU lane remains valid for v0.2.

High-IPC mode expands to multiple integer lanes: ALU0, ALU1, branch ALU, address-generation ALU.

ALU results feed bypass/forwarding network and token scoreboard.

## 8.3 Vector Register File and SIMD Unit

Defines conventional vector registers separate from tensor descriptors.

Supports fixed-width SIMD lanes for integer and floating operations.

Vector state is context-switchable and associated with thread_id/core_id.

Vector results may request FPERR/numeric integrity checks before Aegis-sensitive commit.

# 9. LSU, MMU, and Memory Subsystem

## 9.1 Load/Store Unit

Performs address generation, alignment check, access-size decode, sign/zero extension, and memory ordering control.

Maintains baseline store buffer and future load/store queue.

Reports faults: alignment, page permission, sealed state mutation, Aegis quarantine, COW fault, and bus timeout.

Interacts with RSM for copy-on-write and rollback-aware memory view.

## 9.2 MMU / TLB

Provides virtual memory translation for OS processes.

TLB entries carry ASID/process_id, privilege permissions, cacheability, Aegis region policy, RSM page-root identity, and branch/state ownership.

TLB invalidation occurs on context switch, RSM rollback, sealed-state transition, Aegis quarantine, and page-table update.

## 9.3 Cache Hierarchy

L1I: private per-core instruction cache.

L1D: private per-core data cache with branch/state-aware tags.

L2: optional per-core or cluster cache interface in v0.2; implementation staged later.

L3: future shared SoC cache for octa-core scaling.

Cache lines must encode state ownership and safety metadata when participating in RSM-managed memory.

# 10. RSM-Aware Cache Rules

[x] Cache lines know branch/state ownership

[x] Sealed state lines cannot be mutated

[x] Rollback invalidates speculative cache lines

[x] Commit can promote cache lines

[x] COW faults interact with cache tags

[x] Aegis can quarantine dirty/untrusted lines

Each cache line should eventually include valid, dirty, coherence state, branch_id, state_id, sealed, speculative, aegis_trusted, and quarantine bits.

Baseline v0.2 may implement these as metadata fields and simulation checks before full cache RTL is complete.

A sealed line rejects store hits unless RSM/Aegis explicitly authorizes transition through COW or unseal flow.

# 11. NexusBus / AXI-like Interconnect Specification

NexusBus is the custom internal interconnect family that carries ordinary memory transactions plus Nexus-V identity metadata. It may map to AXI-like channels for FPGA integration, but architecturally it must preserve state, branch, thread, core, and Aegis information.

[x] Valid/ready handshake

[x] Transaction ID

[x] Core ID

[x] Thread ID

[x] Branch ID

[x] State ID

[x] Privilege level

[x] Read/write/atomic op

[x] Burst length

[x] Fault response

[x] Aegis policy tag

| Bus | Purpose |
| --- | --- |
| NexusCoreBus | Core to L1/L2/RSM/memory path. |
| NexusMemBus | L2/L3 to memory controller path. |
| NexusAegisBus | Audit, policy, quarantine, attestation, and security telemetry. |
| NexusTensorBus | CPU/RSM to Tensor/NPU command and result path. |
| NexusTopoBus | CPU/Tensor/RSM to topology fabric route path. |
| NexusQBus | CPU/RSM/Aegis to quantum domain path. |
| NexusDebugBus | Trace, probe, debug, and verification path. |

# 12. Memory Controller System

[x] SRAM/BRAM controller for FPGA prototype

[x] DDR controller interface

[x] RSM page memory controller

[x] Tensor scratchpad memory controller

[x] Cache refill/writeback controller

[x] DMA controller

[x] Secure/Aegis memory region controller

[x] Memory scrubber / ECC controller

The v0.2 memory controller boundary must support simple SRAM/BRAM first because that is easiest for simulation and FPGA bring-up.

DDR/AXI integration comes later through a bridge adapter.

RSM page memory and COW must remain logically above raw physical memory allocation.

DMA is permitted only through Aegis policy and RSM ownership checks.

# 13. Interrupts, Timer, CSR, and Multitasking

[x] Timer interrupt

[x] Interrupt controller

[x] Privilege levels

[x] Context switch support

[x] Per-thread CSR bank

[x] Page-table/MMU support

[x] Process address spaces

[x] Aegis process trust context

[x] RSM branch ownership model

Multitasking requires a precise boundary between OS process, hardware thread, NVRT task, and RSM branch.

Context switch state includes PC, scalar registers, vector registers, CSR bank, state token, branch token, RSM memory root, Aegis context, and pending accelerator jobs.

Timer interrupt is the minimum preemption source for an OS/NVRT scheduler.

# 14. SMT and Hardware Thread Specification

[x] Per-thread program counter

[x] Per-thread architectural register rename/map state

[x] Per-thread state token

[x] Per-thread branch token

[x] Per-thread exception/interrupt state

[x] Shared execution units

[x] Thread scheduler / issue selector

[x] Thread fairness and priority policy

[x] Per-thread commit/rollback rules

[x] Thread context table

[x] Per-thread PC

[x] Per-thread architectural state

[x] Thread scheduler

[x] SMT issue selection

[x] Per-thread RSM state root

[x] Per-thread Aegis context

[x] Thread-aware commit/rollback

| Object | Definition |
| --- | --- |
| Hardware thread | A real SMT execution context inside one core with its own PC, CSR state, architectural map, state token, branch token, and interrupt/trap context. |
| NVRT task | Runtime-scheduled software workload that may map to a hardware thread, tensor job, route job, or quantum job. |
| RSM branch | Versioned computation timeline; it may be owned by a thread but is not itself a CPU thread. |

# 15. Dynamic Sequence Logic and High-IPC Roadmap

The Dynamic Sequence Unit is the future Nexus-V high-IPC brain. v0.2 defines its interfaces; v0.3 can implement out-of-order and SMT behavior incrementally.

[x] Instruction sequencing

[x] Dependency tracking

[x] Issue queues

[x] Reorder buffer

[x] Branch prediction

[x] Speculative execution

[x] State-token dependency rules

[x] RSM-safe retirement

[x] Aegis-safe commit gating

[x] Rollback recovery

[x] Multiple execution lanes

[x] Integer ALU cluster

[x] Branch unit

[x] LSU

[x] Vector/SIMD unit

[x] Tensor dispatch path

[x] Route dispatch path

[x] Quantum dispatch path

[x] Commit/audit path

[x] Instruction fetch queue

[x] Branch predictor

[x] Instruction decoder

[x] Micro-op cache or decoded instruction cache

[x] Rename table

[x] Physical register file

[x] Reorder buffer

[x] Reservation stations

[x] Load/store queue

[x] Memory disambiguation

[x] Multi-port register file

[x] Bypass/forwarding network

[x] Retire/commit unit

[x] Token scoreboard

[x] Branch-state dependency predictor

[x] Route-token dependency tracker

[x] Tensor job completion predictor

[x] Aegis commit-latency hiding

[x] RSM rollback recovery path

[x] Fetch queue

[x] Rename/scoreboard

[x] Issue queue

[x] Bypass network

[x] Token dependency scoreboard

# 16. Multicore and Octa-Core Roadmap

Nexus-V remains single-core for v0.2. The following requirements define the eventual octa-core architecture without forcing premature replication.

[x] Multiple Nexus-V core instances

[x] Per-core program counters

[x] Per-core register files

[x] Per-core state tokens

[x] Shared RSM coherence model

[x] Inter-core interrupt/signaling system

[x] Shared L2/cache fabric

[x] Memory consistency model

[x] Branch/state ownership rules between cores

[x] Aegis per-core attestation IDs

[x] NVRT hardware/software scheduler for threads

[x] Core bus

[x] Memory bus

[x] Aegis bus

[x] Tensor bus

[x] Topology bus

[x] Quantum bus

[x] Debug/trace bus

[x] Transaction routing

[x] QoS / priority

[x] 8 core instances

[x] Shared L3

[x] Shared RSM service

[x] Shared Aegis root

[x] Shared tensor/topology/quantum accelerators

[x] Inter-core interrupts

[x] Cache coherence

[x] Multicore branch/state ownership rules

| Version | Core/thread target | Memory/interconnect target |
| --- | --- | --- |
| v0.2 | 1 core, 1 thread, SMT-ready metadata | L1I/L1D defined, optional L2 interface, NexusBus transaction schema |
| v0.3 | 1 core, 2-way SMT optional, high-IPC pipeline | Load/store queue, token scoreboard, better cache behavior |
| v0.4 | 1-2 cores test cluster | L2/L3/cache coherence prototype, memory controller integration |
| v0.5 | 8 cores, initially 1 thread each; later 2-way SMT = 16 hardware threads | Shared L3, shared RSM/Aegis services, inter-core interrupts, coherence |

# 17. Phase 7 Conventional CPU Core Completion Checklist

[x] Instruction fetch unit

[x] Decoder/control unit

[x] Program counter

[x] Branch unit

[x] LSU

[x] MMU/TLB

[x] L1 instruction cache

[x] L1 data cache

[x] Optional L2 interface

[x] CSR/control registers

[x] Interrupt/timer inputs

[x] Scalar register file cleanup

[x] Optional vector register file

[x] Instruction fetch

[x] Decode/control unit

[x] Program counter

[x] Branch unit

[x] Scalar register file cleanup

[x] LSU

[x] MMU/TLB

[x] L1I cache

[x] L1D cache

[x] CSR file

[x] Interrupt/timer basics

# 18. Phase 8 High-IPC Core Checklist

[x] Fetch queue

[x] Branch predictor

[x] Rename/scoreboard

[x] Issue queue

[x] Reorder buffer

[x] Reservation stations

[x] Bypass network

[x] Load/store queue

[x] Retire/commit unit

[x] Token dependency scoreboard

# 19. Phase 9 SMT / Hardware Threads Checklist

[x] Thread context table

[x] Per-thread PC

[x] Per-thread architectural state

[x] Thread scheduler

[x] SMT issue selection

[x] Per-thread RSM state root

[x] Per-thread Aegis context

[x] Thread-aware commit/rollback

# 20. Phase 10 Cache and Memory Hierarchy Checklist

[x] L1I

[x] L1D

[x] L2

[x] Shared L3

[x] Cache coherence

[x] RSM-aware cache tags

[x] Memory controller

[x] ECC/scrubber

# 21. Phase 11 NexusBus / Custom Interconnect Checklist

[x] Core bus

[x] Memory bus

[x] Aegis bus

[x] Tensor bus

[x] Topology bus

[x] Quantum bus

[x] Debug/trace bus

[x] Transaction routing

[x] QoS / priority

# 22. Phase 12 Octa-Core Scaling Checklist

[x] 8 core instances

[x] Shared L3

[x] Shared RSM service

[x] Shared Aegis root

[x] Shared tensor/topology/quantum accelerators

[x] Inter-core interrupts

[x] Cache coherence

[x] Multicore branch/state ownership rules

# 23. Recommended SystemVerilog Source Set

rtl/core_v02/
├─ nvisc_core_v02_pkg.sv
├─ nvisc_pc_branch_unit.sv
├─ nvisc_instruction_fetch_unit.sv
├─ nvisc_instruction_decoder.sv
├─ nvisc_control_unit.sv
├─ nvisc_csr_file.sv
├─ nvisc_interrupt_controller.sv
├─ nvisc_timer.sv
├─ nvisc_lsu.sv
├─ nvisc_mmu_tlb.sv
├─ nvisc_l1i_cache.sv
├─ nvisc_l1d_cache.sv
├─ nvisc_l2_interface.sv
├─ nvisc_vector_regfile.sv
├─ nvisc_vector_alu.sv
├─ nvisc_token_scoreboard.sv
├─ nvisc_thread_context_table.sv
├─ nvisc_dynamic_sequence_unit_stub.sv
├─ nvisc_nexusbus_if.sv
├─ nvisc_core_v02.sv
└─ tb_nvisc_core_v02_reset_fetch_decode.sv

# 24. Verification Plan

| Test | Purpose | Pass criteria |
| --- | --- | --- |
| v0.2 reset smoke | Core reset and basic CSR/PC initialization | PC reset vector valid, CSR defaults sane, no faults. |
| fetch/decode smoke | Instruction fetch from simple memory image | Fetch valid, decode class correct, illegal instruction not asserted. |
| ALU/regfile smoke | Basic ALU instruction path | Register writeback matches expected value. |
| LSU smoke | Load/store to simple SRAM model | Address generation, write, readback, and alignment fault work. |
| MMU/TLB smoke | Translate mapped and unmapped addresses | Mapped access succeeds, unmapped faults. |
| L1D/RSM smoke | Speculative write and rollback invalidation | Speculative cache line invalidated on rollback. |
| Aegis commit smoke | Aegis audited commit from core path | Unaudited commit denied, audited clean commit allowed. |
| Interrupt/timer smoke | Timer tick causes interrupt/trap path | Timer compare triggers pending interrupt and PC redirect. |
| Vector smoke | Optional SIMD lane add | Vector result matches expected lane-wise output. |

# 25. Final v0.2 Completion Statement

Nexus-V Core Completeness Specification v0.2 is now defined.
The design remains single-core for implementation bring-up.
The architecture is now prepared for SMT, high IPC, cache hierarchy, NexusBus, memory controllers, and eventual octa-core scaling.
The next implementation milestone is to create the v0.2 RTL source set and integrate a minimal fetch/decode/PC/CSR/LSU path into the existing Verilator-passing SoC.