Nexus-V SoC Top-Level Integration and Hardware-Plan Pre-Audit

nexus_v_soc.sv Integration Wrapper - v0.1

# 1. Purpose

This document defines the top-level Nexus-V SoC integration wrapper, nexus_v_soc.sv. It physically instantiates the Phase 0 through Phase 5 hardware blocks and wires the major routing, memory, Aegis, tensor, and quantum boundaries into a single cohesive processor shell.

It also performs a pre-audit against the hardware plan to verify that the SoC integration covers the required capabilities: 3D state-space compute, Git-like reversible state, more-than-binary logic, depth-parallel tensor execution, Aegis safeguard auditing, anti-cheat/privacy proof, topology/polyhedral routing, thermal/error constraints, and hybrid quantum-step support.

# 2. SoC Integration Principle

NVASM / NVIR / NVVM / NVRT define behavior.

Phase 0-5 RTL blocks define hardware units.

nexus_v_soc.sv ties them into one processor boundary.

The SoC wrapper is not yet a final commercial chip. It is an integration scaffold that proves every subsystem has a physical connection path and a testable signal boundary.

## 3. Integrated Phase Blocks

[x] Phase 0 scalar front-end, register file, ALU slice, state token interface, and compatibility route path

[x] Phase 1 RSM rollback controller, delta log interface, and metadata-table read/write boundaries

[x] Phase 1 Aegis Enclave, Commit Gate, ATTEST token path, and MINREP minimal report path

[x] Phase 3 rhombic-dodecahedron topology fabric with route flits, topology tokens, and route trace

[x] Phase 4 Tensor/NPU engine with spatial tensor routes, NPU sync, numeric integrity, and Aegis audit request

[x] Phase 5 Hybrid Quantum Domain with QSIM/QHW mode, QSYNC, QAUDIT, QCOMMIT, and rollback invalidation

# 4. Top-Level Block Diagram

nexus_v_soc

├─ Phase 0 Front-End / ALU / State Token

│  └─ direct ALU_ROUTE and decoded G.ROUTE compatibility path

├─ Phase 3 Topology Fabric

│  └─ SoC-level route flits, route tokens, node masks, route trace

├─ Phase 4 Tensor/NPU Engine

│  └─ 16-node tensor execution mapped over topology fabric

├─ Phase 5 Hybrid Quantum Domain

│  └─ QSIM/QHW, collapse register, qtokens, QSYNC/QAUDIT path

├─ Phase 1 RSM Rollback / Delta Log Controller

│  └─ active root rebinding, descendant invalidation, rollback fault path

└─ Phase 1 Aegis Enclave

└─ commit gate, telemetry/trust score, seal outputs, ATTEST, MINREP

# 5. Critical SoC Wiring Boundaries

| Boundary | Function |
| --- | --- |
| Phase 0 -> Aegis | state token branch_id/state_id/topology_epoch/thermal_epoch contribute to commit and route context |
| Phase 0 -> Phase 3 | decoded G.ROUTE can inject a route flit into the SoC topology fabric |
| Phase 3 -> Aegis | route token validity contributes to Aegis trust and commit checks |
| Phase 4 -> Phase 3 | tensor engine emits route requests for A/B tile movement across the fabric |
| Phase 4 -> Aegis | tensor numeric_ok and npu_audit_req feed policy-sensitive result validation |
| Phase 5 -> Aegis | QAUDIT/QCOMMIT path requires Aegis authorization before quantum results are trusted |
| RSM -> Aegis | rollback availability and RSM integrity feed Commit Gate authorization |
| Aegis -> RSM | seal_state/seal_branch outputs lock committed metadata against mutation |

# 6. nexus_v_soc.sv Interface Summary

| Interface group | Signals / behavior |
| --- | --- |
| Scalar/front-end | instr_valid_i, alu_op_i, domain_i, src/dst register fields, debug register write path |
| Topology | g_route_valid_i, source/target node, route policy, node fault/untrusted/hot masks |
| RSM | rollback_req_i, rollback_state_id_i, rollback status and active pointer updates |
| Aegis | commit request, candidate state hash, policy ID, audit token, telemetry/provenance inputs |
| Tensor | tensor_op_i, tensor IDs, rows/cols/inner_dim, numeric fault injection |
| Quantum | qop_i, qmode_i, qreg index, seed, QHW timeout/fidelity simulation inputs |
| Status | soc_status_o, commit_allow/deny/refuse/quarantine, MINREP valid, ATTEST valid |

# 7. SoC Source Files

| File | Purpose |
| --- | --- |
| integration/nexus_v_soc_pkg.sv | Top-level status struct and SoC fault enum |
| integration/nexus_v_soc.sv | Master SystemVerilog wrapper instantiating Phase 0-5 modules |
| integration/tb_nexus_v_soc_smoke.sv | Smoke test harness for commit, topology route, tensor, and quantum paths |
| integration/filelist.f | Compilation-order filelist for all phase packages/modules plus SoC wrapper |
| docs/pre_audit_against_hardware_plan.md | Machine-readable capability audit summary |

## 8. Top-Level Integration Checklist

[x] nexus_v_soc_pkg.sv exists

[x] nexus_v_soc.sv exists

[x] Phase 0 front-end is instantiated

[x] Phase 1 RSM rollback path is instantiated

[x] Phase 1 Aegis enclave is instantiated

[x] Phase 3 topology fabric is instantiated

[x] Phase 4 Tensor/NPU engine is instantiated

[x] Phase 5 Hybrid Quantum engine is instantiated

[x] Aegis commit outputs are exposed at SoC boundary

[x] Aegis seal_state/seal_branch outputs are generated

[x] RSM rollback and Aegis rollback availability are connected

[x] Tensor route requests feed the Phase 3 topology fabric

[x] Quantum QAUDIT/QCOMMIT path is connected to Aegis authorization

[x] SoC status object reports active branch/state/commit, route/tensor/quantum progress, and fault class

# 9. Pre-Audit Against Hardware Plan

| Hardware-plan capability | SoC integration coverage | Verdict |
| --- | --- | --- |
| 3D state-space compute | Phase 3 fabric + Phase 0 state token + Phase 4 tensor spatial mapping | Satisfied at SoC boundary |
| Git-like branch/rollback/commit states | RSM rollback controller + Aegis commit gate + state/branch IDs | Satisfied for Phase 1 integration |
| More-than-binary logic | Phase 0 domain enum + encoded MVL + Tensor/Quantum domains | Interface satisfied; physical MVL is future work |
| Depth-parallel tensor compute | Phase 4 16-node Tensor/NPU engine uses topology routes | Satisfied |
| Aegis safeguard auditing | Aegis Enclave telemetry inputs, commit/refuse/quarantine outputs | Satisfied |
| Anti-cheat/privacy proof | MINREP path + I/O/GPU/HID provenance telemetry inputs | Satisfied at hardware-hook level |
| Topology/polyhedral routing | Phase 3 rhombic-dodeca fabric, route flits/tokens/masks | Satisfied for rhombic profile |
| Error/numeric integrity | Tensor numeric_ok + Aegis numeric_integrity_ok + RSM integrity inputs | Satisfied at signal level |
| Thermal constraints | node_hot_mask, thermal_safe, topology/thermal epoch fields | Satisfied at signal level |
| Hybrid quantum domain | Phase 5 QSIM/QHW engine + QSYNC/QAUDIT/QCOMMIT wiring | Satisfied |
| State explosion control | Runtime-level design exists; SoC exposes status/fault/branch IDs | Partially satisfied; hardware branch scheduler future work |
| Merge semantics | NVIR/VPHI covered; SoC has state/commit paths | Partially satisfied; full VPHI hardware future work |

# 10. Known Integration Limits

The SoC wrapper intentionally does not solve final silicon-level complexity. It proves architectural connectivity. The following items remain future deepening tasks:

true clock-domain crossing and asynchronous FIFOs

full RSM metadata SRAM instances wired into all controllers

complete branch budget scheduler in hardware

full VPHI merge resolver hardware

real cryptographic signatures/MACs replacing placeholder token logic

full ECC/parity arrays across memories

physical thermal sensor and power-governor fabric

alternate topology generators beyond the rhombic-dodecahedron profile

synthesis cleanup and simulator-specific package conflict resolution

## 11. SoC Smoke-Test Checklist

[x] Clean Aegis commit path asserts commit_allow or ATTEST valid

[x] Decoded G.ROUTE injects a topology flit into Phase 3 fabric

[x] Tensor TMATMUL path emits route request and produces tensor status

[x] Quantum QALLOC/QPREP/QH/QMEASURE path produces observable quantum progress

[x] RSM rollback path accepts rollback request and exposes active-root update/fault outputs

[x] SoC status reports fault class rather than raw subsystem-specific fault only

[x] MINREP path remains privacy-safe and does not expose raw memory/private fields

# 12. Implementation Notes for Compilation

The source ZIP preserves each phase in a separate directory. Several earlier phase packages intentionally define similar concepts, especially route policy enums. The SoC wrapper uses package-qualified names and casts at boundary crossings. If a simulator reports duplicate package/type conflicts, compile the latest package for each family only or merge common type definitions into a single shared package in the next cleanup pass.

# 13. Final Integration Rule

Nexus-V SoC is valid only when:

- state can branch and roll back,

- topology routes carry branch/state identity,

- tensor/quantum results synchronize through state tokens,

- Aegis can refuse unsafe commits,

- MINREP can prove integrity without exposing private memory,

- and all committed state passes the Aegis Commit Gate.

# Appendix A. nexus_v_soc.sv Excerpt

`timescale 1ns / 1ps

// Nexus-V SoC top-level integration wrapper.

// This module is an integration skeleton that physically instantiates the Phase 0 through Phase 5 blocks

// and wires the major route, state, RSM, Aegis, tensor, and quantum boundaries together.

// It is intentionally conservative: final arbitration, clock-domain crossing, and full issue/retire logic

// remain Phase 6+ work.

import nexus_v_soc_pkg::*;

module nexus_v_soc #(

parameter int NUM_NODES = 16,

parameter int HASH_W    = 128

)(

input  logic clk,

input  logic rst_n,

// Phase 0 scalar/front-end execution control.

input  logic                 instr_valid_i,

input  nvisc_pkg::nvisc_alu_op_e alu_op_i,

input  nvisc_pkg::nvisc_domain_e domain_i,

input  logic [4:0]           src_a_i,

input  logic [4:0]           src_b_i,

input  logic [4:0]           dst_i,

input  logic                 dst_we_i,

input  logic                 dbg_r_we_i,

input  logic [4:0]           dbg_r_waddr_i,

input  logic [nvisc_pkg::XLEN-1:0] dbg_r_wdata_i,

// Decoded G.ROUTE / G.GEO_PATH input.

input  logic                 g_route_valid_i,

input  logic [15:0]          g_source_node_i,

input  logic [15:0]          g_target_node_i,

input  logic [2:0]           g_route_policy_i,

// RSM rollback input.

input  logic                 rollback_req_i,

input  logic [31:0]          rollback_state_id_i,

// Commit/Aegis input.

input  logic                 commit_req_i,

input  logic [31:0]          candidate_state_id_i,

input  logic [31:0]          candidate_branch_id_i,

input  logic [31:0]          candidate_commit_id_i,

input  logic [HASH_W-1:0]    candidate_state_hash_i,

input  logic [15:0]          policy_id_i,

input  logic                 audit_token_valid_i,

input  logic [HASH_W-1:0]    audit_token_subject_hash_i,

// Telemetry and provenance inputs for the Aegis trust aggregator.

input  logic                 memory_hash_ok_i,

input  logic                 rsm_integrity_ok_i,

input  logic                 branch_lineage_ok_i,

input  logic                 io_provenance_ok_i,

input  logic                 hid_chain_ok_i,

input  logic                 gpu_path_ok_i,

input  logic                 thermal_safe_i,

input  logic                 voltage_safe_i,

input  logic                 rollback_available_i,

input  logic                 numeric_integrity_ok_i,

input  logic                 tamper_detected_i,

input  logic                 safeguard_bypass_i,

// MINREP request.

input  logic                 minrep_req_i,

input  logic [31:0]          session_id_i,

// Phase 4 tensor/NPU command path.

input  logic                 tensor_instr_valid_i,

input  nvisc_tensor_pkg::nvisc_tensor_op_e tensor_op_i,

input  logic [31:0]          tensor_a_id_i,

input  logic [31:0]          tensor_b_id_i,

input  logic [31:0]          tensor_c_id_i,

input  logic [15:0]          tensor_rows_i,

input  logic [15:0]          tensor_cols_i,

input  logic [15:0]          tensor_inner_dim_i,

input  logic                 tensor_inject_numeric_fault_i,

// Phase 5 quantum command path.

input  logic                 quantum_instr_valid_i,

...

# Appendix B. Pre-Audit Source Basis

This document was aligned against the hardware plan that defines Nexus-V as a 3D, dense state-space processor with more-than-binary logic, reversible/Git-like branches, depth-parallel compute, Aegis safeguard auditing, anti-cheat/privacy-proof requirements, thermal/error constraints, and alternative polyhedral processor shapes.